Sinclair ZX Spectrum Emulator 'Z80' v3.05  -  11/11/96  -  by G.A. Lunter

Contents:

1.    INTRODUCTION, REGISTRATION, GENERAL INFORMATION

1.1   Some general remarks

1.2   Registration

1.3   Other emulators

1.4   Acknowledgements

1.5   Disclaimers

1.6   Copyright notices

2.    THE EMULATOR

2.1   Starting the emulator

2.2   Using the emulator

2.3   Keyboard emulation

2.4   Screen emulation

2.5   Sound emulation

2.6   Loading programs from tape

2.7   Using .TAP files

2.8   Using .VOC files

2.9   Multi-load games

2.10  Using the Microdrive

2.11  Using the Disciple and Plus D interfaces

2.12  Using the Multiface

2.13  Using the RS232 channel

2.14  On joysticks and mice

2.15  The utility ConvZ80

2.16  Converting file formats - the utility CONVERT

2.17  The utilities Z802TAP, TAP2TAPE and TAP2VOC

2.18  The utility OUT2VOC - and how to make .OUT files

2.19  The utilities READVOC and READSB

2.20  Quick overview of command line switches, and features

2.21  Miscellaneous remarks

3.    THE SAMRAM

3.1   Basic extensions

3.2   NMI software

3.3   The built-in monitor

4.    THE SPECTRUM

4.1   The Spectrum

4.2   The Interface I

4.3   The DISCiPLE and Plus D Interfaces - Introduction

4.4   The DISCiPLE and Plus D Interfaces - Basic commands

4.5   The DISCiPLE and Plus D Interfaces - More advanced commands

4.6   The DISCiPLE and Plus D Interfaces - The snapshot button

4.7   The DISCiPLE and Plus D Interfaces - The system variables

4.8   The Spectrum 128

5.    TECHNICAL INFORMATION  (in file TECHINFO.DOC)

5.1   The Spectrum 48K

5.2   The Spectrum 128K

5.3   The AY-3-8912 sound chip

5.4   The ZX Printer

5.5   The Interface I

5.6   The SamRam

5.7   The Multiface 128

5.8   The AMX mouse interface

5.9   The Z80 microprocessor

5.10  File formats

1. INTRODUCTION, REGISTRATION, GENERAL INFORMATION

1.1  Some general remarks

This is the documentation for 'Z80', a Sinclair ZX Spectrum 48/128

emulator.  The program turns your PC into a Spectrum.  Its features in

a (largish) nut-shell:

- Emulates ZX Spectrum 48K model 2 or 3, and Spectrum 128K.

- Best compatibility of current Spectrum emulators.

- Emulates Interface I and Microdrive (cartridge in cartridge file),

Disciple and Plus D disk interfaces, Multiface 128.

- Full tape support: loads both normal and turbo-saved programs using

either tape interface on LPT port, SoundBlaster AD converter or

.VOC sound sample files; can also use .TAP binary tape image files

by trapping ROM save/load routines.  Saving back to tape is also

possible, either directly to tape or to a .VOC sound sample file.

- Full emulation of Spectrum screen, including all colour effects.

Border, and timing-sensitive border and attribute effects are also

emulated.

- Emulation of ZX Printer, for Epson and HP Laserjet compatible

printers.

- Emulation of beeper through PC beeper, and of 128K sound through

either PC beeper or AdLib (SoundBlaster) compatible cards

- Supports digital and analogue joysticks, and mouse, controlling

Cursor, Sinclair, Kempston or user-defined joystick.

- Emulates Spectrum AMX Mouse interface (used by e.g. Art Studio)

- Runs on any PC, including oldies with 8088/8086 processor.

- Emulator runs just above 100% of actual Spectrum speed on 20 MHz

286 and 25 MHz 386SX; can be slowed down on faster machines.

- Z80 emulation includes emulation of R register, all unofficial

instruction codes, and unofficial flags; interrupts once every

1/50th emulated second; I have tried to make each instruction

execute in a time proportional to the time taken on a real Z80.

- Registered package includes all source files.

- Interface I and Spectrum 128's "p" channel input/output can be

re-routed to LPT or COM port, or to a file.

- Includes several utilities; to convert from and to other snapshot

and tape file formats, to convert snapshots into tape files, tape

files into sound sample files, to display contents of snapshot

files, to convert screen snapshots to .PCX and .GIF files, to read

.VOC sound sample files from the LPT tape interface or

SoundBlaster.

- A program is included which makes .VOC or raw sound sample files

of sounds produces by the Spectrum, either by the beeper, the

soundchip of the Spectrum 128, or the MIC port.

- OUTs to any port can be logged, and stored in a log file with

timing information.  This log file can be translated into a .VOC

sound sample file using OUT2VOC, for both ear and AY soundchip

output.

- Simple tracing feature

- Opcode mapping feature, tags bytes that are executed during

emulation; useful for debugging and disassembling purposes.

(It is meant to be used with the disassembler program DNSA by

Leslie Styles, lms@soton.ac.uk)

- Multi-level support as in XZX 0.5.2; also support of .SLT files.

- Emulator can set a breakpoint, invisible to the running program.

- Emulator can load and save blocks of Spectrum memory to disk

directly.

- Emulator can run under Windows (95).

- Supports VGA, EGA, CGA and Hercules video adapters including

monochrome VGA.  (Support for Plantronics discontinued.)

There is quite a lot to explain in this documentation.  First of all

the emulator itself requires some of your PC's resources.  It is not

really a demanding program, but there are some things that need

attention.  These technicalities are dealt with in section 2.1.

Some general things about the emulator are explained in section 2.2.

After reading sections 2.1 and 2.2, you will know most of the basics.

The details can be found in subsequent sections.

The Spectrum has a number of ways to communicate with the outside

world, like the obvious keyboard and the screen, but also the

microdrives, the DISCiPLE/Plus D disk interfaces, the tape interface,

the beeper, the sound chip of the Spectrum 128, the Kempston joystick,

the AMX Mouse, the ZX Printer interface, and the RS232 channel of the

Interface I and Spectrum 128 can be used to communicate with PC

channels in some way. For instance, the keyboard is connected to the PC

keyboard, and the tape I/O can be routed to a file, as well as to a

physical tape recorder, or it can come from a .VOC sound sample file.

All these things are explained in the rest of chapter 2.

For our own Spectrums Johan Muizelaar and I built a piece of hardware

we called the SamRam (which has nothing to do with the SAM Coupe, by

the way!).  It contains a monitor program and software to make

snapshots of programs.  It's still very useful and I still use it a

lot.  An explanation of its functions is to be found in chapter 3.

Some things peculiar to the Spectrum or its interfaces, not specific to

this program but useful to know, are collected in chapter 4.  It

contains for instance a table of Spectrum keywords and the key

combination to get them; unfortunately this information is not printed

on standard PC keyboards! More importantly, in this chapter it is

explained how to use the Microdrive and DISCiPLE interfaces, both of

which are probably unfamiliar to many former Spectrum users.

There are some interesting technical facts about the Spectrum that I

discovered while debugging the emulator.  As much as I could think of

is contained in the final chapter, which can be found in the separate

file TECHINFO.DOC.  You don't need to read this chapter to use the

emulator.  A specification of the file formats used by the emulator is

also included there.

Please note that the Tatort BBS, which used to be the 'Spectrum

emulator support BBS', has ceased to exist.

There is by now a great lot of Spectrum-related information and

software available on the net.  A good starting point is the FAQ

(Frequently Asked Questions) list originally started by Marat

Fayzullin, and now maintained by Damien Burke.  It can be found at

http://www.cs.bham.ac.uk/~dmb/speccy/faq.html.  It is also available at

ftp://ftp.nvg.unit.no/pub/sinclair/docs/specfaq.  This latter site

carries a lot of Spectrum snapshot and Sinclair-related information.

Highly recommended!  A long list of FTP sites carrying Spectrum

programs is included in Damien Burke's FAQ list.

If you have access to internet newsgroups, take a look at

comp.sys.sinclair, where many Sinclair enthousiasts meet and share

thoughts.  You will find lots of well-informed people there.  Please

read the aforementioned FAQ file before asking questions, but be

invited to join in afterwards.  Note: Please do *not* post binaries to

the comp.sys.sinclair newsgroup!

1.2  Registration

'Z80' is a shareware program.  The program is not completely

functional, and the parts which are left out are included when you

register.  You are encouraged to give this demo version to friends, but

DO NOT change the original archive in any way, please.  The shareware

version of the emulator consists of the Z80-304.ZIP archive file, which

should contain the following 12 files:

Z80.EXE      - The emulator itself.

Z80.INI      - Default initialisation file (used by Z80.EXE).

Z80.PIF      - Program Info File to run 'Z80' under Windows

Z80.ICO      - Windows icon.

Z80.DOC      - Documentation file for the emulator.

Z80FAQ.DOC   - Frequently asked questions and answers.

REGISTER.DOC - This file.

NEW.DOC      - The What's New file.

ROMS.BIN     - Various ROM images (used by Z80.EXE).

TECHINFO.DOC - Technical info about the Spectrum etc.

LAYOUT.SCR   - Keyboard layout help screen (used by Z80.EXE).

DIAGRAM.Z80  - Circuit diagram for tape interface, and calibration

The shareware version of the emulator differs from the registered

version in the following respects: it displays a message when the

program is started, and will only run normally for five minutes, after

which the program cannot be slowed down anymore.  Furthermore, it

cannot load from or save to tape, and it does not emulate the DISCiPLE

and the Plus D interfaces.  Everything else works as in the registered

version.

If you register, you get the fully working version, and the following

utilities:

CONVERT  - a general conversion program: can list out BASIC and

translate it back, produce .GIF or .PCX files from

screen dumps, translate Spectrum ASCII (CR) to PC ASCII

(CR/LF), and some other things.

CONVZ80  - Translates various snapshot and tape formats of other

Spectrum emulators into each other.  Can handle the

familiar .SNA format use by several emulators (JPP,

XZX,...), and also Pedro Gimeno's (VGASPEC and SPECTRUM)

.SP format and Kevin J. Phairs' (SPECEM) .PRG format.

It can also handle tape files of SPECEM and L. Rindt and

E. Brukner's emulator ZX.

DISCIPLE - Reads DISCiPLE and Plus D diskettes, both 3.5'' and

5.25''.  It translates the 48K and 128K snapshot files

to .Z80 snapshots, and ordinary files and screen

snapshots to .TAP tape files. (Not necessary for the

fully registered version, but handy for the cheaper

one.)

ADDDAT   - Utility to edit the additional data part of .SLT

snapshot files, which contain loading screens and level

data.

Z802TAP  - Converts a .Z80 snapshot, 48K or 128K, to a .TAP file

which can be loaded into the emulator and saved to tape

by the next utility:

TAP2TAPE - Saves the contents of a .TAP file back to tape, to load

it into an ordinary Spectrum.

TAP2VOC  - Converts a .TAP file to a .VOC sound sample file, to

write to tape, or to load into the emulator.

READVOC  - Reads in a long, 'digital' .VOC sound sample, to be used

as input to the emulator, from the LPT tape interface or

a SoundBlaster.

READSB   - Reads .VOC sound samples from SoundBlaster using DMA,

and applies a digital filtering and oversampling algo-

rithm tuned for Spectrum tapes.

OUT2VOC  - Converts .OUT log files into .VOC or raw sound sample

files, so that you can easily extract music samples from

Spectrum/Spectrum 128 games, or SAVE directly to a .VOC

or raw sample file.

Z80DUMP  - Shows the header and the contents of a .Z80 file.

You will also receive the source files of the emulator, the above

utilities and the SamRam, and you will be kept informed about future

updates.

You can also choose to register for the emulator without DISCiPLE and

Plus D emulation, if you are not interested in those parts.  The

registration fee is a bit lower then, but you will receive everything

stated above (including the DISCIPLE program); the only difference is

that you get a version of the emulator that will not emulate the M.G.T.

interfaces.

There are several registration sites, the main one being B.G. Services

in the U.K.  You can also register with Jimaz in the Czech Republic,

or David Pomeroy in New Zealand, whichever is most convenient.  Please

note that Friendware in Spain and Sinclair Freakeren are no longer

registration sites.

IMPORTANT NOTE:

When sending registrations, please either PRINT your order and address,

or use BLOCK CAPITALS to aid readability.  If you use a postal cheque

service, or direct transfer, please try to also send a letter to the

site with details of the payment etc.  Many payment slips are

unreadable copies.

B G Services, U.K.

------------------

The registration fee is BP 20 or BP 15 (BP=British Pound) for the

version with or without DISCiPLE/Plus D emulation respectively.

Payment can be by cheques (Sterling on a UK bank, or Eurocheques) or UK

postal order made payable to B G Services.  You can also transfer

directly to the Giro account 324.82.16  (B G Services).  Girobank plc,

Bootle, Merseyside G1R 0AA.

You can also send money in cash.  Please send British Pounds and no

other currencies!

Please do not use other currencies than British Pounds on cheques; it

is not economic to change.  If it is the only option available, please

add BP 8 to cover drawing costs.  I regret I cannot accept orders via

charge or credit card.

The address is:

B G Services

64 Roebuck Road

Chessington

Surrey KT9 1JX

(United Kingdom)

Telephone enquiries on (0181) 287 4180, Fax (0181) 391 0744, or from

abroad: +44 181 287 4180 and +44 181 391 0744 respectively.

B G Services can also supply ready built tape interfaces for BP 14.50.

These are professional quality items built into 25W 'D' connectors.

(If this archive is old, please enquire on price before ordering)

Files will be on a 3.5" HD disc together with some ready converted

software, which is either PD, or has the approval of the authors.

Other disc sizes may be requested, but these may not contain all the

extra files.

Myself, The Netherlands

-----------------------

Registration via B G Services is fastest, and is preferred.  In some

cases, however, sending money to The Netherlands may be more convenient.

Dutch users can transfer NLG 50,- (full version) or NLG 35,-- (without

Disciple emulation) to the following Giro account:

59.45.263  (G.A. Lunter, Groningen)

The registration fee of BP 20/BP 15, US$ 30/US$ 25, DM 50/DM 40, can

also be sent, in cash, to the following address:

Gerton Lunter

P.O. Box 2535

NL-9704 CM  Groningen

The Netherlands

Any other (convertible) currency of an approximately equivalent amount

will also be accepted.  From Spain and Italy, postal money orders work

best.

Eurocheques can also be accepted.  Please do NOT use other currencies

than Dutch Guilders (NLG), and do NOT fill in the place you live, but

write 'Groningen' or leave blank.  Cheques that are filled in

differently cost HFL 15-25 to draw.

Jimaz, Czech Republic

---------------------

For registrations in the Czech Republic, you can contact Jimaz.

JIMAZ s.r.o.

Hermanova 37

170 00 Praha 7

phone: +42 2 379 498

fax:   +42 2 378 103

Email: vondrack@jimaz.cz

Registration fee is 500 and 650 Czech crowns.

David Pomeroy, New Zealand and Australia

----------------------------------------

For registrations in New Zealand and Australia, contact

David Pomeroy

P O Box 2939

Shortland Street

Auckland

New Zealand

Phone: (09) 627-9618

Email: davidpom@iconz.co.nz

The registration fee is 40/50 NZ$, or 30/40 AUS$.

==================

All sites reserve the right to ignore any order which does not comply

with the conditions above.

If you have an older registered version and wish to upgrade, please

contact your registration site, sending return postage.  Updates within

the same version number and normally sent free if a disc and return

postage is received.  Updates from previous versions will carry a small

update fee.

1.3  Other emulators

I used to have a list of other emulators for the PC and other computers

here, but the list in Marat Fayzullin/Damien Burke's FAQ was much more

complete and detailed, so I deleted mine.  It lists:

14 emulators for PC's,

6 for the Commodore Amiga,

4 for the Acorn Archimedes,

4 for UNIX platforms,

3 for the Macintosh,

1 for the Atari ST,

1 for the NeXTStep, and

1 very old one for the Commodore 64

I believe this makes the Sinclair ZX Spectrum the most often emulated

computer ever!

There are also emulators available for the ZX81.  Carlo Delhez, who

also wrote a Spectrum emulator for the QL, wrote the ZX81 emulators

XTricator (for the QL) and XTender (for PC's).  There are probably

more.  For more information, take a look at the FAQ of the newsgroup

comp.sys.emulators.misc.

1.4  Acknowledgements

From the very first beginning in november 1988, when I wrote the first

lines of code for the emulator, Johan Muizelaar has been a very

demanding and critical user, being only satisfied when it was perfect.

Besides, there are quite a few things I would never have started

working on if he hadn't insisted that I would!

I have also profited much from the fine cooperation with Brian Gaff,

who, besides handling the UK registrations, also generates a continual

stream of suggestions, remarks and bug reports.  He also brought me

into contact with many people that contributed to the emulator in

several ways.

A major part of the things new to version 3 of the emulator have been

written by Hugh McLenaghan.  He wrote all code for the Disciple/Plus D

emulation, and for the ZX Printer emulation (where I subsequently

introduced bugs...)  Also, Hugh wrote much of the documentation for the

Disciple and Plus D interfaces in this file.

Finally, many thanks are due to

o   Carlo Delhez for information on the '128 and several other things,

o   Andre Mostert for some more '128 info and info on EMS memory,

o   Walter Prins for many '128 programs,

o   Marco Holmer for making the program such a big hit at the HCC dagen,

o   Henk de Groot, for finding a solution to a bug in A86 v3.22,

o   Arnt Gulbrandsen for a suggestion which made the emulator faster,

and information on a group of unofficial Z80 opcodes,

o   Ruud Zandbergen for his digital joystick interface,

o   Jan Garnier for providing the chips to reanimate my real Spectrum,

o   Ettore de Simone for finding a noisy bug,

o   Rudy Biesma and Tonnie Stap for providing info on the DISCiPLE disk

formats,

o   Burkhard Taige for various bug reports on it,

o   Ian Cull for enhancing the DISCiPLE program and two bugfixes,

o   Bert Lenaerts for information on the AZERTY keyboard,

o   Chris Lemon for fixing a bug in the CALL instruction,

o   Robert Zmyslowicz, Einar Gattoni Saukas, Bernhard Lutz,

Erik Kunze and Mr. Lee Tonks aka Blood for their thorough

beta-testing of v3.05.

and all the people that I forgot to mention.

1.5  Disclaimers

The last version contained a few bugs, but this version is guaranteed

to be bug-free.  You can safely operate nuclear power plants or life

support systems with this software.  And if it still happens to break

down, feel free to sue me.

Many of the bugs that disfigured the previous version are gotten rid of

now.  Most notably, the problem that saving a Disciple/+D snapshot

caused old data to be overwritten has been fixed.  On the other hand,

FORMATting Disciple/+D disks still has its problems, but, depending on

the machine in question, they have overcome in particular cases.

Many people have sent me bug reports, for which I am very grateful.  I

have removed most of those, but have not been able to remove them all,

sometimes because the report was too vague and/or irreproducible on my

machine, or because of a mixture of reasons like: difficult to find,

not a problem to most people, easily circumvented, or laziness on my

part.

1.6  Copyrights etc.

Amstrad still holds the copyright on the 48K and 128K rom.  However,

they have kindly allowed free use of them.

The Disciple ROM is (c) Bruce Gordon / Format Publications.

The Plus D ROM is copyright Datel / Format Publications.

Multiface and Lifeguard are registered trademarks of Romantic Robot.

They are licensed from Romantic Robot for use in Z80.

The registered version of the Spectrum emulator 'Z80', and the version

distributed with PC Format, may NOT be further distributed.  The source

codes, which are part of the registered package, may NOT be used in

other Spectrum emulators running on PC's.  In principle, it is okay to

use the Z80 emulator code in an emulator for another Z80-based

computer; however, please do contact me if you want to use it.

2.  THE EMULATOR

2.1  Starting the emulator

The emulator will work on any PC with 640K memory and at least a 80286

processor, with a VGA, EGA, Hercules or CGA video adapter.  If

available, it will also use EMS memory, an Adlib compatible soundcard,

the SoundBlaster card, a mouse, and an analogue or digital joystick.

The emulator will first read in the switches that are given in the

Z80.INI file.  You can enter switches there just like you do on the

command line.  Lines starting with a % sign are ignored; they are used

for comments.

After any switches, you may specify a snapshot file on the command

line. This file will then be loaded and executed directly.  The

extension .Z80 is not necessary.  The emulator will also read .SNA

files (the snapshot format of, amongst others, Arnt Gulbrandsen's JPP);

you don't have to convert them to .Z80 files (but you may want to to

save disk space), and it reads .SLT files (containing snapshots plus

additional data, mostly level data for games consisting of various game

levels).

The emulator tries to figure out what hardware is available, and uses

things as it finds them.  Most of the time this will work without you

having to tell it anything, but if you have to, you can override the

defaults by putting switches on the command line.  Switches that you

use often can be put in the Z80.INI file.  If you give a switch a

second time, for instance if it is also in the Z80.INI file, it will

disable it again.

If you're using a Trident VGA with version 3 BIOS, you may see the

picture compressed at the top of the screen, while the bottom half

contains vertical white lines.  This is due to a bug in the Trident VGA

Bios.  Start the emulator with the switch -xv to get a full picture.

Some VGA adapters do not produce a stable picture when Z80 is using the

standard VGA mode.  If your monitor doesn't sync, try switch -xv.

Some black-and-white VGA monitors only display one of the three RGB

colours (green most of the times), resulting in several Spectrum

colours becoming indistinguishable.  Use -xb to use grey tones instead

of colours.

If you are using Hercules, try starting the emulator with -xh on the

command line.  The emulator will use a non-standard Hercules mode to

display a full-screen Spectrum picture.  You may need to calibrate your

monitor to make the image steady.

If you're using Plantronics, try -p and -q to see which gives the best

result.

If you haven't got EMS memory, the page swapping of the Spectrum 128

cannot be emulated exactly, and, more seriously, it is extremely slow.

Although most programs will work, they will be too slow to be of any

use.  Also, the emulator needs lots of base memory if no EMS memory is

present; if you don't have enough, try specifying -xt on the command

line to make the emulator use as little memory as possible (by

shrinking several buffers).  The emulator uses 332K or 572K of base

memory (with and without EMS memory respectively), and 47K less in both

cases if -xt is specified.  If this is still not enough, try to use

-xu, which saves 83K, but then Hi Resolution Colour emulation does not

work anymore.

On 386 and 486 machines you can emulate EMS by software using EMM386

for instance.  The speed of the EMS emulator determines in part the

emulation speed of Spectrum 128K programs, so it may be wise to try a

few for the best results.  I use QEMM, which seems to be faster than

EMM386.

The Spectrum 128 has a built-in sound chip.  If you have an Adlib

compatible soundcard installed, the Spectrum 128 sound will be played

through the Adlib card.  If you haven't, the loudest of the three sound

channels will be played through the internal PC speaker.  Sometimes the

effect is quite nice, sometimes it is horrible, but it's all I can do

on a standard PC.  If you don't want to have the Spectrum 128 sound

played through the internal speaker, use the switch -xi.  If you don't

want the Adlib card to be used (for instance to hear the sound through

the internal speaker) use -xa.

If you're using the Pro-Audio Spectrum 16 sound card, do not install

the resident FM.EXE program; it causes problems with the emulator. Do

make sure that MVSOUND.SYS is installed in your CONFIG.SYS file, to

make the Pro-Audio Spectrum 16 Adlib compatible.

The noise channels of the Spectrum 128 sound chip can work on different

frequencies, whereas the FM chips of the Adlib card cannot.  However,

if your Soundblaster is equipped with CMS chips, the noise frequency

can be programmed.  Specify -xc to use the CMS chips.  (These chips are

not available on Soundblaster Pro cards, and neither on most

Soundblaster clones).

If you're living in Belgium or France, you are probably using an AZERTY

keyboard.  Specifying -xz on the command line will make all letter keys

and many punctuation keys work in the right way.

If the emulator erroneously detects an analogue or digital joystick,

use the switch -kk.

The emulator can now also be run under Windows 3.1!  However, you

cannot use the tape interface and Real mode doesn't work anymore.  In

version 3.04, keyboard handling under Windows is much improved, and

works as well as without Windows.  If you let the emulator run

full-screen you may use EGA or VGA, if you want to run it windowed

you'll have to use CGA, because the virtual video display driver of

Windows cannot handle the VGA mode I use (although it's only a standard

text mode; even Windows 95 doesn't know how to handle it). You'll

probably want other default settings of some parameters (such as the

video mode) if you run the emulator under Windows; the emulator will

always use the .INI file in the directory of the Z80.EXE file so the

other switches must be put on the command line or in a .PIF file. An

example .PIF file (which runs the emulator in windowed CGA mode) and a

.ICO icon file are supplied.

The emulator measures the speed of the computer it runs on in the

normal way.  Under Windows 95 this gives reliable results; however,

Windows 3.1 causes Z80 to run about 3 to 4 times slower than normal.

You may have to tinker with the speed setting to get it to run at a

reasonable speed.

The emulator will automatically detect whether Windows is running, and

act appropriately.  To run the emulator in Windows compatibility mode

in a normal DOS environment, use -xw.  If you use this switch under

Windows, it turns windows compatibility mode OFF, and hangs unless you

use Hi Resolution Colour emulation (which doesn't rely on the timer).

When running the emulator under Desqview, use -e for EGA mode display.

At present running under OS/2 is not a good idea as it will crash if

the speed is altered.

These are the most important switches that you have to specify when you

start the emulator.  Most of the other switches are used to select

default values for various things which can also be changed when the

emulator is running.  Some useful things to select are default

directories for .Z80, .TAP and .MDR files; these will be explained

below.

2.2  Using the emulator

In this section, the basic functions of the emulator, residing under

the function keys F1-F10, are explained.

When the emulator starts, you'll see the usual Spectrum copyright

message appear on screen.  Pressing F1 will pop up a small help screen

that explains the function of the function keys and various other

special keys.

By pressing F10, you enter the main menu of the emulator.  Most of the

menu options can be chosen directly by pressing another function key; a

small help screen pops up if you press F1.  If you're somewhere deep in

the menu structure from the main menu, pressing ESC will get you one

level higher most of the time.  Pressing F10 will get you back to the

main menu.

The 'Select Hardware' menu option sits under function key F9.  There

are seven major configuration to choose from:

Spectrum 48K

Spectrum 48K + Interface I

Spectrum 48K + SamRam + Interface I

Spectrum 48K + M.G.T. Interface  (i.e. DISCiPLE or Plus D)

Spectrum 128K

Spectrum 128K + Interface I

Spectrum 128K + M.G.T.

This menu also allows to choose the M.G.T. type, +D, Disciple with a

pre-loaded operating system supporting graphics output using the Epson

format, and one supporting HP-PCL format graphics output.

In all configurations except the one with SamRam, the Multiface 128 can

be emulated too.  The Multiface 128 software is aware of, and can read

and write to, the Microdrive of the Interface I and the Disciple and +D

disk drive.

After a change has been made, pressing ENTER switches to that mode and

resets the Spectrum.  If you don't want the Spectrum to reset, pressing

CTRL-ENTER will switch to the new mode while preserving as much of the

runninge program as possible.  Switching from 128K mode to 48K mode

will almost always crash the program, except if you enter the SPECTRUM

command before switching.  Be careful when you change the disk drive

emulation; when switching an MGT interface (Disciple or +D) on or off,

it is wise always to reset the emulator, as otherwise the hardware will

often not match the system variables.

To use SamRam's monitor on a 128 program, switch the hardware from the

main menu, and generate an NMI (Extra functions - N) before returning

to the emulator.  This will often work, but you can't return to the

program without crashing it.

On a real Spectrum 128, the menu bar of the startup screen is moved

using the cursor keys on the '128 keyboard.  These keys simultaneously

press a normal cursor key (5,6,7 or 8) and shift.  So you can shift the

menu bar with shift-6 and shift-7.  It is possible to use the PC cursor

keys for this; you have to select Cursor joystick emulation (which is

selected by default) and press Num-Lock once to have the PC-cursor keys

press the Spectrum Shift key too.  You could also specify -xs on the

command line (or put it in the Z80.INI file) to make the PC cursor keys

by default press shift for you in '128 mode; see also the Miscellaneous

remarks section.

The Save and Load Program options (F2 and F3) will save the whole state

of the Spectrum and some of the emulators' settings to a .Z80 snapshot

file.  It will pack the data somewhat, so that the length of the file

varies.  The amount of memory saved depends on the current hardware

mode; 48K for normal Spectrum, 80K for SamRam, and 128K for Spectrum

128.  (Note that the RAM contents of the M.G.T. interface or the

Multiface 128 are not saved.)  The settings that are saved are those

that are program dependent, for instance which joystick emulation is

used, and more technical settings like those of the R register, LDIR

and Issue 2 emulation and video synchronisation.  These are explained

below.

Loading a .Z80 file will cause several settings to be changed.

Resetting the Spectrum will not reset these settings to their default

values!  Especially the joystick emulation setting change can be

confusing.

All settings can be checked and changed in the Change Settings menu,

which pops up if you press F4. You can do many things here.  The I and

O options can be used to redirect the RS232 output; see section 2.13

for information on this.  R - R register emulation, and L - LDIR

emulation are usually only necessary when loading programs;  for

remarks on these options see section 2.6, and section 5.1 for more

technical details.  Other settings and switches are:

H - Hi resolution colour emulation, also called 'coppering'.  To

eliminate flickering of moving characters, and to see some colour

effects otherwise not visible.  Also, this allows you to see the famous

loading stripes in the border.  See section 2.4 for more information.

2 - Issue 2 emulation will turn the emulated Spectrum in an Issue 2

Spectrum.  (This option also works, but is out of place, in Spectrum

128 mode).  Some very old programs (Blue Ribband, Spinads) will not

respond to the keyboard properly on Issue 3 Spectrums, and for these

programs this option was added.  Seldom needed.

S - sound enables you to turn off all sound, useful for late-night

playing.

F - toggle between 50 Hz frame interrupt rate (the standard value) and

some value defined via the switch -dN, setting the frame rate to 100/N

Hz.  Without switches, the alternate rate is 100 Hz.  If -dN is used

the default rate is set to 100/N Hz and alternate rate to 50 Hz; is -dN

is specified twice (with the same divisor N) then default and alternate

rate are swapped.

Doubling the interrupt frequency is useful for slow machines, as

some programs will run faster when this option is on.  If you're typing

in a BASIC program on a slow machine, always turn this on, since the

keyboard, which is polled by an interrupt routine, will respond much

better.  On fast machines, this feature is useful for running Spectrum

programs quickly while keeping a normal keyboard response.  Some

programs will crash with a frame interrupt rate different from 50 Hz.

V - video synchronisation is used to remove the flickering of moving

characters in some programs.  You can choose between Normal, High and

Low.  Normal works well for almost all programs; Ghosts and Goblins and

Zynaps look much better when this is turned to High.  If you see

characters not moving smoothly or flicker, or a background not moving

as a whole, experiment a little bit with this setting, and re-save the

snapshot when you've found the best setting.  On fast computers, try to

use Hi res colour emulation instead.  This setting has no effect when

Hi res colour emulation is on.  (For a more detailed discussion of this

option see section 2.4 and section 5.1)

J - joystick emulation specifies which Spectrum joystick the PC cursor

keys (and mouse, and analogue or digital joystick, if available)

control. You can choose from Cursor (default), Kempston, Interface 2

and user-defined.  As already said above, if Cursor joystick is chosen,

the Num-Lock key controls whether Shift is pressed too with a joystick

movement. (Since the shift and number keys are pressed exactly

simultaneously, it is possible that the Spectrum has already read the

Shift key, but not yet the others, when you press both keys down.

Sometimes you will therefore get the number 5,6,7 or 8 instead of a

cursor movement. If you have used a +3 or +2A Spectrum, you will be

familiar with this!)

Finally, C - Change speed lets you control the speed of the emulator.

As a side effect, slowing down the emulator makes the timing of the

various opcodes correspond more exactly to the actual timing on a real

processor.  (Remember this is not possible on slow PCs!)

That concludes the discussion of the F4-'change settings' menu.  Let's

continue with the other function keys.

F5 generates an NMI. This is used to activate the Samram, Multiface

128, or start the Disciple/+D Snapshot facility. If none of these are

active it may reset the Spectrum or do nothing.

ALT-F5 or CTRL-F5 resets the Spectrum.

F6 turns on Real Mode.  Try this when the emulator is playing a tune

and sounds a little harsh.  This mode is needed when you want to load

turbo-saved games from tape; see below for more information.

F7 and F8 activate the tape and Microdrive/M.G.T. menus.  Again, see

below for more information.

Resetting the Spectrum, or generating an NMI can be done from the main

menu too, in the X - Extra Functions menu.  This is useful if you want

to activate the NMI software of the SamRam for instance just after

loading a snapshot file, or just after you changed the hardware mode.

In this menu it is also possible to save or load a memory block or

screen snapshot; to set a breakpoint (see section 2.21) or to

temporarily shell to DOS. Furthermore, here you can find another

sub-menu for the OUT logging feature.  If port FE is logged, such a log

file can be translated into a .VOC sound sample file using the OUT2VOC

utility.

When you're typing BASIC-programs in 48K mode, you'll probably have to

look up some Spectrum keywords.  Further down in this documentation

there is an alphabetical list of all keywords and their

key-combination. For 'on-line' help, press ALT-F1 to see the Spectrum

keyboard layout.

2.3  Keyboard emulation

The keyboard.  Letter keys are mapped to the Spectrum's letter keys.

The ALT and CTRL keys can both be used for Symbol Shift.  Then, there

are a lot of keys on the PC keyboard which don't exist on the Spectrum

keyboard.  Many of them are used, to make things easier:

The function keys have several special functions.  See the previous

section.

CTRL-Break and CTRL-ALT-DEL quit the emulator.  Better use F10-Q-Y

though.

The punctuation keys - = ; ' , .  / and their shifts: _ + : " < > ?

have the effect of pressing Symbol Shift and the corresponding letter

key, so you can use these in the straightforward way.

The ESC key presses Shift-1, EDIT, used as a sort of ESC key in many

Spectrum programs.  The Backspace key presses Shift-0, the Delete of

the Spectrum.  CapsLock presses Shift-2, Spectrum's capslock key.

The PC-cursor keys and the numeric keypad keys 8,4,6 and 2 control the

Cursor, Interface 2, Kempston or user-defined joystick.  The TAB key,

and 0,5 and ./DEL on the numeric keypad control the fire button.  If

the Cursor joystick is selected, you can select whether Shift should

also be pressed with the NumLock key (but see the discussion above of

the -xs switch).

If you're running the emulator on a slow computer, try doubling the

interrupt frequency (switch -d).  Most programs poll the keyboard by

interrupt, in any case the ROM does, and doubling the frequency with

which this happens will make the emulated Spectrum react much more

quickly on your keystrokes.

If you've got an AZERTY keyboard, the standard mappings of the keys

won't work at all properly.  Include the switch -xz in your Z80.INI

file in this case; many punctuation keys will now also work properly.

There is no support for other non-US keyboard layouts; sorry!

You can re-map the keyboard by yourself by using the -kx switch.  It

has the format

-kxABCD

where #AB denoted the scan-code (in hex, between #01 and #58) and #CD

the key that should be pressed when the emulator sees this scan code.

Normal keys are coded by bytes below #40; the lower three bits denote

the row number (0-7), and the upper three bits the column number (1-5).

Column 1 of rows 0-7 are respectively Shift,A,Q,1,0,P,Enter and Space.

Column 2 to 5 are the keys on the same line, but more toward the

center.  For instance, C is on row 0, column 4, and has code 0+8*4=32=

#20.  Function key F11 has scan code #57, so to map F11 to C use

-kx5720.  Code keys '#CD' in the range #C0 - #DD are used to code for

special keys.  Here is the list of codes in this range:

#C0  Edit                          #CF  Joystick left

#C1  - or _                        #D0  Joystick right

#C2  + or =                        #D1  + or =

#C3  Delete                        #D2  Joystick down

#C4  ; or :                        #D3  Joystick fire

#C5  " or '                        #D4  F1

#C6  , or <                        #D5  F2

#C7  . or >                        #D6  F3

#C8  / or ?                        #D7  F4

#C9  *                             #D8  F5

#CA  Capslock                      #D9  F10

#CB  Numlock (Shift toggle)        #DA  F6

#CC  Scroll lock (+CTRL=Quit)      #DB  F7

#CD  Joystick up                   #DC  F8

#CE  - or _ (for grey -)           #DD  F9

Simultaneous keypresses on the PC keyboard are only transmitted to the

PC to a certain extent, depending on the particular keyboard.  My old

keyboard could handle TAB and the keys on the numerical keypad

simultaneously, but my new keyboard can't: when I press TAB and hold it

down, further key presses on the numeric pad are not seen by the PC

at all.  There is absolutely nothing to be done about it.  Playing

games can be a problem sometimes.  You'll have to live with it.  All

information sent by the keyboard is used.

2.4  Screen emulation

There are two different ways the emulator can emulate the screen.  The

standard way, in which 50 times an emulated second the screen is

checked for changes, and they are subsequently displayed on the

monitor. This works fine, and was the only mode available in previous

versions. It works basically the same with all video modes; however,

some are much slower than others. EGA is notoriously slow, due to the

ugly way it has to be addressed. VGA is definetely the fastest. EGA and

VGA are the only video modes in which the colour of the overscan can be

controlled; in these modes, some emulation of border effects (loading

stripes, for example) can be done. You don't see these effects in the

other video modes.

Changing the overscan colour results in 'snow' lines appearing on the

screen.  It is possible to eliminate these by waiting until the video

adapter is in horizontal retrace.  This takes some time, therefore the

emulator doesn't do this in Real Mode. Real time loading requires

minimum delay, so the screen does not update. You can select the update

option, but you risk Tape Loading errors!

In the standard mode, the point (relative to the 50 Hz interrupt) at

which the screen is displayed can be controlled by setting the 'Video

Synchronisation Mode' to normal, high or low.  If you see moving

characters flickering excessively, try changing this setting.  For

instance, BC's Quest for Tires won't look at all good with a wrong

setting; many programs display subtle differences in different modes.

In the Hercules and CGA modes not all colours can be displayed.  In EGA

mode, all colours can be displayed, but some colours have the same

intensity in Bright 1 as in in Bright 0.  Only in VGA mode the colours

resemble closely the Spectrum originals.

The other video emulation mode is called Hi-resolution colour emulation

mode. In this mode great care is taken to display each video line at

exactly the right time, and also the precise times the border colour is

changed is used to build the border pattern. In this mode, every

hi-resolution colour effect is visible; all flickering problems are

also eliminated (provided they weren't already there on a real

Spectrum!), so no need to adjust the Video Synchronisation Mode.

The drawback of using this mode is that it is much slower, since the

emulator has to keep track of the number of T states passed, and also

has to grab a line from screen memory and put it in a buffer 192 times,

50 times a second.  But on fast computers, 486's and up, the emulator

can still easily emulate at 100%.

HRC mode only works with a VGA video adapter.

Finally, when loading turbo-saved programs in Real mode, screen updates

are suspended because they take too much time (even on fast computers).

You can refresh the screen by pressing U - Update in Real mode, but you

should do this only if you're sure that the emulated program is not

loading a block, for pressing U during loading is a sure way to get a

tape loading error.

2.5  Sound emulation

The Spectrum beeper is emulated by the PC's internal beeper.  Because

every 1/50th of a second the screen has to be updated, and this takes a

little time even if there are no changes, the sound is a bit harsh.  If

you select real mode, the emulator won't update the screen and music

will sound good.  If you select HRC mode, the sound will be even

harsher than it does in normal mode.

The sound of the Spectrum 128's sound chip is played through the Adlib

card; if you haven't got such a card some notes are played through the

internal speaker.  That sound will be turned off, however, as soon as

the program makes a sound through the normal speaker of the Spectrum.

Some Spectrum 128 programs use the sound chip and the beeper at the

same time, and this won't work properly without an Adlib card.

2.6  Loading programs from tape

This emulator can load programs that are saved to tape in the usual

way, but also turbo-saved programs can be loaded.  Furthermore, you can

make a disk file act as an 'emulated tape', so that the normal SAVE and

LOAD commands can be used to transfer data to and from disk easily.

The emulator can load programs from .VOC sound sample files.  This is

very useful for multi level games; the levels on the tape can be put in

.VOC samples, and loaded when needed without having to re-install the

tape recorder.  How to use .VOC files is described in section 2.8

below.  The current section is about loading programs directly from

tape.

First of all, you need an interface to connect the tape recorder to the

PC.  There are two ways of doing this.  You can use a tape interface on

the LPT port.  A circuit diagram is in the DIAGRAM.Z80 program, or you

can obtain a ready-made interface from B G Services, see section 1.2

for more details.

Version 3 can also use the SoundBlaster for tape input.  This is easier

since no additional interface is needed.  The LPT tape interface

however is more robust in loading.  When using the SoundBlaster, you

have to be careful not to put the volume level too high, since

otherwise the SoundBlaster will clip, which is a sure way to producing

tape loading errors.  The LPT tape interface is not sensitive to this.

It has proven very difficult to load programs directly via

SoundBlaster.  This is due to the fact that the emulator does not have

time to filter the SoundBlaster input (except for a trivial hi-pass

filter to remove a 50 Hz hum) in Real mode.  For direct loading, the

LPT interface is strongly recommended.  For loading via the

SoundBlaster, the utility READSB (see section 2.19) will read in and

filter the sample.  Loading via READSB is, I think, as reliable as

loading via the LPT interface.

If you want to save programs back to tape, you have to use the LPT

interface; it is not possible to use the SoundBlaster for this.  (There

is a way to save programs in standard format to tape though; put them

in a .TAP file - see below - convert it to a .VOC file and play this

sample to a tape.  Use a good sample player, one that doesn't halt for

half a millisecond between blocks!)

In the program DIAGRAM.Z80 a tape tester is present.  The LPT tape

interface has to be calibrated, and this program can also be used to

check the volume level when using the SoundBlaster as input device. The

variable resistor of the LPT tape interface has to be adjusted so that

the bar, which should go to 0% when the volume is down, points just

below 50% at normal volume.  In the case of the SoundBlaster, when the

volume is turned up, the bar first goes from 0% to 50% and over, and

then saturates and drops below 50% again.  The volume has to be set

such that it is just 50% in the non-saturated zone.

You have to tell the emulator which LPT port you use for tape I/O.  Use

command line switch -b2 for example to select LPT2 for tape input (or

put it in the Z80.INI file).  Use -xo to select the SoundBlaster for

tape input.  It uses base address 0220 by default; if this is not

correct, use for instance -xq 0240 to specify address 0240.  The

emulator does not use the SoundBlaster interrupts.

There are two ways to load programs: in 'real' or normal mode.  In real

mode, the emulator doesn't update the screen or scan the keyboard, so

that the emulated Spectrum program can run smoothly.  The emulator has

to run at about 100%, but then you're able to load everything a normal

Spectrum would load, including turbo-saved programs.  The only thing

you see on screen are the loading bars in the border (on EGA or VGA

screens).  Real mode is selected by pressing F6.  Saving programs in

real mode is a bit useless but it works; enter the SAVE command, press

a key to start saving and quickly press F6 when the saving starts.  It

will continue in real mode.

Please note that in real mode, the PC clock is not updated.  It will

run behind for as many minutes as real mode has been switched on.  The

CMOS clock won't be affected, so the next time you start up your

computer things will be normal again.

If your computer is just fast enough, don't slow the emulator down too

much.  Because the IN instruction is relatively slow, the emulator has

to run at about 110% for the best results.  If your computer is really

fast, you can best slow it down to exactly 100%.  If your computer is

just a bit too slow, you can try to make your tape recorder run slower

too (usually you can do this by adjusting a little screw at the back of

the motor), I successfully loaded several speed-saved programs at 90%.

In normal mode, the standard ROM loading and saving routines are

'trapped' (at addresses 04d8 and 056a) when they're about to start

saving or loading.  A routine in the emulator itself then takes over,

and loads or saves a block to tape (or a disk file, see below).

When you enter LOAD "", the emulator starts loading using its internal

loading routine.  You'll see a blue window appear.  Pressing F6 now

will switch to Real mode, and the emulated Spectrum program will

continue the loading process (if it is running at 100%).

Every time a block is loaded or saved, a window containing some

information of the block to be loaded or saved appears.  If you don't

want this, for example because you want to enjoy the loading screen,

specify -tx on the command line.

Using these SAVE and LOAD routines has a great advantage as well as a

disadvantage compared to using the Spectrum's own routines in real

mode. The advantage is that the internal routines work on every

machine, no matter how slow or fast, without having to make the

emulator run at 100%.  The disadvantage at using them is that they

obviously won't understand turbo-saved files.  For normal use, these

internal routines work much easier, and real mode loading is only

necessary for turbo-saved and well protected programs.

Today, most programs are protected.  The emulation of the Z80 processor

has to be exactly right, or those strange decoding routines that use

all features of the processor the programmer could think of, will

definitely crash.  It is most important to switch the R-register and

LDIR emulation on, as virtually all protected programs use at least the

R register.  Sometimes programs are real sensitive to the timing of the

interrupts; if programs refuse to load, try sampling the first few

blocks in a .VOC file (see below) and load it from there (of course

with R register and LDIR emulation on); when loading from VOC files,

interrupt timings are exact.

2.7  Using .TAP files

The emulator uses files with the extension .TAP to hold a piece of

'tape', with several blocks on it.  Each block is usually either a

header or a data block; a normal file thus consists of two blocks.

There are two modes of operation when loading and saving to disk files,

single and multiple .TAP file mode.

In single .TAP file mode, each block saved is appended to the end of

the .TAP file, like what would happen if you were actually saving to

tape.  In the same way, when loading in single file mode, each time the

ROM wants to load a block, it is presented the next block in the .TAP

file.  It is handled as it would if the block was loaded from tape, so

that if the ROM needs a header and is presented a data block, it will

skip it.  The header will however be considered to be read.  So,

entering LOAD "rubbish" will show all headers in the .TAP file, just as

an actual Spectrum would show all headers on the tape if you left the

tape running.

If the last block is loaded, the file pointer is moved to the start

again.  So a .TAP file can be considered to be an infinite tape. Single

.TAP file mode is useful to save whole programs to disk, or for

multi-load games that need to load in levels as you play.

A sort of 'random access' file management is also useful, for instance

when you're developing a program and need to save several pieces of

data to disk and later load back a specific one.  This can be done in

single .TAP file mode (by positioning the file pointer using the Browse

function), but there's a different mode of operation that makes things

easier: multiple .TAP file mode.  In fact, by default the emulator is

in this mode.

When the emulator is in multiple .TAP file mode, it will read all

blocks from all .TAP files in a specified directory, one after the

other.  When it has finished reading the last one, it will start all

over again.

When saving, the emulator will put the two blocks of a normal file, the

header and the data block, in one .TAP file with a unique name made up

of the printable letters of the file name and a two-digit number.  The

name of the .TAP file is irrelevant to the emulator, but to have it

resemble the name of the actual Spectrum file you saved is simply

convenient.  If the Spectrum program saves a data block to tape without

first saving a header, the .TAP file produced will contain only this

data block, and the DOS file name will be HDRLES, with a two-digit

number appended to make it unique.  The format of the .TAP files saved

in multiple .TAP file mode is exactly the same as the format used in

single .TAP file mode.

You can easily string together .TAP files; for instance a number of

.TAP files created in multiple .TAP file mode can be put into one big

.TAP file simply by concatenating them, e.g.

COPY /B FILE1.TAP + FILE2.TAP ALL.TAP.

Now you know what you can do, but how to get the emulator to do it?

That's what the final section is about: the tape menu.

Press F7 to enter the tape menu.  Pressing S will select or de-select

single file mode.  As a default, multiple .TAP file mode is selected.

In this mode there are three other possible choices in the menu.  First

of all, D selects a tape-file directory where the .TAP files will be

saved into and loaded from.  A default directory can be selected by

putting the -xs switch on the command line or in the Z80.INI file; for

example -xs c:\spectrum\taps.

The I and O options are used to select the source and destination of

the saving and loading: the LPT port (in/out) or SoundBlaster port

(only input) for a physical tape recorder, or 'disk' for disk files.

Input and output are directed to disk by default if a default tape file

directory is given by means of a switch on the command line or .INI

file.

If Single .TAP file mode is selected, different and more menu options

appear.  With R and W, the input and output tape files can be selected.

They may be the same.  If a specified output file already exists, you

may choose to append to or overwrite this old file.  Saving is always

at the end of the file; loading always starts at the beginning of the

.TAP file.

A useful shortcut is ALT-F7: it will automatically select single file

mode, and prompt for an input .TAP file.

With the B option - Browse - the position of the file pointer into the

input .TAP file can be changed.  If you, for instance, type LOAD""

instead of LOAD "" CODE, the first header is read, and you would have

to read all other headers before trying to load the file again.  With

the browse option you can conveniently change the file pointer.  Of

every header (that is, every block with flag byte 0 and length exactly

17) the name and type, and of every data block the length is shown.

The option B can also be used to delete specific blocks from a .TAP

file.  Make sure you do not only delete a data block or a header, or

the ROM may get confused.  (Double data blocks will be skipped, but

double headers can generate Tape Loading errors).

As in multiple .TAP file mode, I and O are used to specify the source

and destination for saving and loading.  If you enter a .TAP file name

with R or W, this will automatically be set correctly.  You can then

always reset the input or output back to physical i/o again, of course.

Finally, in Single .TAP file mode you can use 'tape mirroring': loading

programs from tape (in normal mode, i.e.  not using Real mode) and at

the same time saving a copy of each block loaded into a .TAP file. This

.TAP file can later be used to load the program again, in case

something goes wrong.  There are two ways of mirroring: normal

mirroring and exact mirroring.  The last one must be used only in

exceptional cases; it will always make a copy of a block, even if it

had a tape error (the corresponding block in the .TAP file will also

have a tape error).  This causes ticks in leader tones to make 0-byte

blocks, so the .TAP file may get messy.  Do not use exact mirroring if

you don't really have to; I think normal mirroring will always work in

practice.

If you try to leave the tape menu when for instance tape mirroring is

selected, and no output filename is given, the emulator will warn you

and will insist that the error be corrected.  Yes, it's stubborn!

One final point about multiple .TAP file mode.  If you select a

directory that contains no .TAP files at all, and try to load from the

directory, the emulator will reset itself to physical tape I/O.  Select

a different directory and try again.  Note that simply putting a .TAP

file in the directory does not reset the emulator to multiple .TAP file

mode, as it will not look in the directory again as soon as it uses

physical tape I/O.

2.8  Using .VOC files

.VOC files contain sound samples, so they can also contain the sound of

a computer tape.  The emulator can load programs from these sound

sample files, by keeping track of exactly how much time elapses during

the emulation of a program, and every time the emulated Spectrum

program reads the EAR port, supplying it with the right sample.  Via an

intermediate file, a .OUT log file, the emulator can also SAVE and

produce a .VOC file with the right sounds in it.

In this section .VOC file playback is discussed.  For a discussion of

how to SAVE to a .VOC file, or how to produce a .VOC file of

Spectrum-generated music, please read section 2.18 about the OUT2VOC

utility.

Because the emulator has to keep track of the emulated time, and also

has to do some calculation every time the EAR port is read (which a

program does rather often when loading), the emulation speed drops

considerably (by a factor of two, roughly) when playing back .VOC

sample files.  So loading programs using VOC files may take longer than

loading them directly.  On the other hand, it is not at all necessary

to have the emulator run at 100% now, so that even slow computers can

load turbo-saved programs, provided that they can read samples.  And on

fast computers, the emulator can be set at the maximum speed, to load

programs much faster than usual.

Using .VOC files is sensible for instance when you have a multi level

game, where subsequent levels are saved in a non-standard format, so

that they cannot be stored in .TAP files.  The higher levels can be

stored in .VOC files, and loaded when needed, without having to

re-install the cassette player.

You can take a sample with any sampler program.  The emulator supports

most VOC block types.  It does not support compressed blocks, though;

use full 8 bit samples.  The problem with some samplers is that they

leave a small gap between subsequent blocks within the .VOC file.

These gaps are usually inaudible, but do cause tape loading errors.  It

is very important that the sample is taken without any gaps.  The

READVOC program reads samples from the LPT tape interface or the

SoundBlaster, and can take long continuous samples without gaps.

Because the emulator converts the 8-bit sample to a simple High or Low

value, READVOC also uses only two values for the signal height.  This

results in highly compressible .VOC files; compression factors of about

25 are no exception.

READVOC is good at taking samples from the LPT tape interface, but it

performs worse when used to take samples from sound cards.  You'll be

better off using samplers provided with the card.

It is also possible to convert .TAP files to .VOC files, with the

utility TAP2VOC.  This was useful to test the .VOC playback feature,

and also makes it possible to write .TAP files back to tape using a

sample player.

To play back a VOC file, press F7 to go to the tape menu, and press P

to select the VOC file playback menu, and enter the name of the VOC

file to be played. The VOC file is now in stand-by. Just for

convenience, the emulator starts playing back the VOC file as soon as

the ROM loading routine is hit; that is, as soon as you type LOAD ""

and ENTER.  The emulator will now play the VOC file to the end

(irrespective of what the emulated Spectrum program is doing with the

sound supplied to the EAR port!).  When the end is reached, a window

will pop up, and the emulator switches back to normal emulation mode.

It is possible to pause the play-back at any time.  Do not do this

while loading, since it will immediately result in a tape loading

error.  It is also possible to start play-back without LOAD "", which

is necessary for instance when the VOC file consists of a turbo-saved

block.

And finally, it is possible to wind and re-wind the VOC file, as if it

were a tape.  The position is displayed in minutes, seconds and

hundreds of seconds.

Starting the emulator by "z80 -tv file.voc" will play that .VOC file to

the EAR input as soon as the emulator starts.  It makes sense also to

load a .Z80 snapshot file from the command line.  To make a snapshot

file that starts loading right away, type LOAD "" <ENTER>, press F6

(real mode), B (back to normal mode), F2 (save snapshot) and save it.

2.9  Multi-load games

Some games have several levels saved on the tape, to be loaded when a

previous level is completed.  There are several ways to handle these

programs.

First, simply playing it, and loading next levels using a real

tape-recorder seems a good solution.

If the data blocks in which the level data is saved are written to tape

in the standard format, you can convert them to .TAP files.  It is most

likely that the program uses the usual ROM routine to load the blocks,

and this routine is trapped by the emulator, so that instead of loading

from tape, the blocks can be loaded from these .TAP files.  To convert

the blocks to .TAP file, use a standard tape-to-tape copy program and,

in the Tape menu (F7), specify that tape output should go to some file.

If the blocks are just code blocks with header, simply enabling the

Tape Mirroring option and loading the blocks at 16384 (ignore the

crashes that most probably result; just reset the emulator and

continue) will do the trick.  If the blocks are headerless, then go to

the SamRam, press NMI (F5), D for Disassembler, and L:4000 (with tape

mirroring enabled) to load and mirror each block.  Here also, ignore

possible crashes which result if the block is too long.

A general method is to store the level data blocks in .VOC sample

files.  These files are huge, lengths of several megabytes are common,

but they can be compressed very tightly (if you used READVOC or READSB

to make them).  Then, if you need a block, simply shell to DOS,

decompress it, and load it.

Finally, if you want to hack into the program, you can also try to find

the routine that is responsible for the loading of a level.  At the

appropriate point, enter an ED FB instruction, with A containing the

level number, and HL the address at which the block is to be loaded.

The emulator will then try to load the data.  If loading failed, the

carry flag is toggled; it is untouched if the data was loaded

successfully.

The level data can be stored in a .SLT file (Super Loader Trap file),

which contains a snapshot and can contain data for several levels.  If

the current program was not loaded from a .SLT file, the emulator will

look for files with the .DAT extension.  The name is made from the

snapshot last loaded, with the required level number appended in

decimal; letters are dropped from the file name if necessary to make

them at most 8 characters long. If the file is not found, the user is

informed of the level number, and given the opportunity to specify a

name.

.SLT files can be produced using the ADDDAT utility.  See also section

5.10, file formats, for more information.

This ED FB feature first appeared in XZX 0.5.2, and was devised by

Russell Marks.  It is now supported by many other emulators as well.

2.10  Using the microdrive

Compared to the tape, this is really simple.  Cartridges are emulated

by files of 137923 bytes.  These files have the extension .MDR, and can

contain up to 126K of data.  The emulator emulates 8 microdrives, the

maximum amount the Interface I software can handle, and each of these

cartridge files can be inserted in any of the 8 microdrives.  (Do not

insert one file into more than one microdrive; this will cause problems

with the buffering done by the emulator as well as the Interface I, and

might result in data loss).

Press F8 to enter the microdrive menu.  Press 1 to 8 to select a

microdrive, and I to insert a microdrive cartridge.  You can select an

existing one, or type a new name.  If the cartridge file isn't found,

the emulator asks whether it should create it.  When created, you'll

have to format it first; if you don't, you'll get a 'microdrive not

present' error when you try to read it, just as happens with real

unformatted cartridges.  To format a cartridge, type

FORMAT "m";1;"name"

After this the cartridge should have 126K of free space.

The cartridge can be write protected; see the menu option in the F8

menu.  This is a characteristic of the cartridge, and the write protect

tab information is therefore stored in the cartridge file.

As on the real Spectrum, you'll have to be careful with OUT's if a

cartridge is inserted.  Try OUT 239,0 (on a real Spectrum, this turns

on the microdrive motor) and wait a few seconds; most of your data will

be lost!  You can stop the microdrive motor by typing STOP (or, more

generally, generate an error).

The microdrives are emulated at IN/OUT level.  This means that every

utility or program that uses microdrives ought to work on the emulator.

Most utilities use hook codes, and these will certainly work.

The GAP line is emulated; this signal is activated if the interface I

senses a piece of tape with no data on it.  If the checksum of the

first header block of a microdrive header or data block is not correct,

that block is considered to be a GAP.  This will only happen if some

utility writes a bad block to microdrive deliberately, if the file is

newly created and unformatted, or when you type OUT 239,0.

To try the microdrive, load a Spectrum program, switch to Spectrum

48K+If.1 mode if necessary, and turn on Multiface 128 emulation.  Press

F8, I(nsert cartridge), enter some file name and create an unformatted

cartridge, return to the Spectrum and press F5 (NMI), S(ave), enter

some name, choose M(icrodrive), choose F(ormat) and wait a few seconds,

then choose S(ave).  Then reset the Spectrum, and type LOAD

*"m";1;"name" to re-load the snapshot you just made.

Alternatively, switch to SamRam mode, make sure a formatted cartridge

is present, press F5, S, M, S, name, and press ENTER twice to have the

SamRam save a snapshot to microdrive.  This snapshot can only be loaded

in SamRam mode.

2.11  Using the DISCiPLE and Plus D interfaces

For an explanation of the commands of the DISCiPLE and Plus D

interfaces, see the relevant sections in chapter 4.

The ROM of the DISCiPLE is supplied with the emulator, together with

two version of the D.O.S. pre-loaded in RAM.  The ROM of the Plus D

interface is not supplied, for copyright reasons.  The emulator will

only emulate a Plus D interface if it can find a file PLUSDROM.BIN in

the emulator's own directory.  If you have a Plus D yourself, you can

transfer the ROM in the following way.  Load the operating system from

a Plus D system diskette, and save the entire ROM, together with the

operating system, to disk by entering SAVE d1"rom" CODE 0,16384.  Then,

start the emulator and choose hardware mode Spectrum 48K + Disciple

(not Plus D of course), then type LOAD d1"rom" CODE 32768 to load the

rom at address 32768.  Then type

POKE 48623,0

to reset the 'open file' counter; failing to do so will result in data

being overwritten when you use the Plus D snapshot feature as the Plus

D system will not build up a used-sector-bitmap if this counter is

non-zero.  You can check that this counter is 1 by typing PRINT PEEK

48623; this one open file is the file "rom" itself.

Finally, press F10, X, S for Save Data block, set start address and

length to 32768 and 16384 (decimal) respectively, and save the ROM by

the name PLUSDROM.BIN.  If you re-start the emulator now, it will

emulate the Plus D too.

(Aside: to use your own version of the DISCiPLE rom, proceed as above,

but type

POKE 40431,0

Note that for this, the base of DISCiPLE's system variables must be

#0298 = 664 decimal.  If it is different, change above address

(corresponding to location #1DEF but displaced by #8000 bytes, i.e.

relative position 6999 decimal).  Then use DEBUG or any flexible

file cut-and-paste utility to put the 16K file in the correct position

in the ROMS.BIN file; see file TECHINFO.DOC for the exact position.

End of aside.)

The emulation of the DISCiPLE or Plus D interfaces in turned on simply

by choosing the right hardware mode in the F9 menu.  Changing the

M.G.T. type will result in a Spectrum reset (unless you change with

CTRL-ENTER), since a different ROM is switched in.

By default, disks 1 and 2 refer to drives A: and B: respectively, but

this can be changed in the F8 menu.

2.12  Using the Multiface 128

The following is an excerpt from the original Multiface 128

documentation:

MULTIFACE is a registered trademark af RORANTIC ROBOT.  Multiface 128

is a true multi-purpose interface with

1) fully universal and 100% automatic back-up facility for tape,

microdrive, Discovery, Plus D or Disciple (although the Discovery is

not emulated, so that that feature is not usable),

2) 8K RAM extension - suitable for GENIE, LIFEGUARD, or as a buffer,

3) MULTI TOOLKIT to study/modify/develop programs, POKE infinite lives,

etc.

Multiface 128 works on the Spectrum 48/128.  It can be used any time in

any mode; it is immaterial what is inside the Spectrum at that moment

or how or from what source (tape, disk, cartridge) it has been loaded

(or typed in, etc.).  Multiface does not save programs, but computer

contents (compressed RAM image).  Upon returning from the M128 or upon

re-loading, the programs continue as if never frozen.  To use the M128:

1) Push the NMI key (F5)

2) Select the required option from the menu

The MAIN MENU options are:

Exit: to abandon the Multiface and exit to BASIC (provided BASIC is

present).  By using exit, you come out of the Multiface entirely.

All efforts are made to preserve the current program in the Spectrum

intact.  The main condition is the existence of standard system

variables - without these the Spectrum crashes.  A successful exit

gives you full access to the program.  To restart it, if needed, you

must know the starting line or address.  Exit is impossible in the

128K mode when the edit ROM is paged and Spectrum ROM is off; in such

cases it does not appear on the menu at all.  All in all, you can

only exit to BASIC if it exists - it is like jumping into a pool: a

joy if it's full of water, a crash if there is none.

Return: to continue the program.

Save: to proceed to the SAVE routines:

a) input the name of the program.  Up to 9 characters (or just press

ENTER to the input RUN automatically as a default)

b) save to: Tape, Microdrive, Opus [does nothing], Disciple (use D

for Plus D too).  You can save the entire program (including the

screen), or the screen only.  Programs are compressed to take the

minimal room possible and to load faster.  Screens saved on their

own are left intact as standard Spectrum screens.

c) Format microdrive cartridges.  Cartridges are automatically named

after the programs to be saved.

Tool: to access the MULTI TOOLKIT routines.

quit - to return to the main menu

Enter - to PEEK and scroll through addresses or to POKE

Space - to allow you to type in a new address

hex - to toggle between hexadecimal and decimal display format

reg - to point to the Z80 registers as they were when the program was

frozen

window - to open a window with full on-screen editing using the

cursor keys.  The flashing window address corresponds to the

address in the bottom edit line.  The window display is by default

in hex, but you can change it to

text - to see the 128 bytes in the window as ASCII text.

select - to inspect RAM banks 0-7 in 128K mode.  Press s + the bank

number

Print - to dump screen to printer.  For printer interfaces using COPY

command.  You can POKE address 8200 (decimal) with the following

values, if you wish to

113 to turn the line feed on (cr+lf)

112 to turn the line feed off (cr)

17 to dump screen as text with the line feed on

16 to dump screen as text with the line feed off

Jump - not to return, but jump to another address.  Strictly for

machine code users only.  Enter the address to jump at 8192/3

(low/hi).  You can jump to Spectrum ROM/RAM and to M128 8K RAM.  As

the M128 overshadows the ZX ROM (8192-16383), address 8194 determines

the paging status: if it is 0, the M128 RAM remains paged, 1 pages

out the RAM and any other value disables the jump command completely.

[Note: if it is 0, also the M128 ROM (0-8191) remains paged.]

You can jump from the main menu, and you can also pre-program M128 to

jump directly upon NMI (F5) and by-pass the M128 ROM software

entirely.  To program the direct jump, POKE 8192-3 with the jump

address, and then also 8195-7 with a special identification word RUN

(i.e. 82,85,87).  Whenever you press NMI now, you will jump to the

predefined address and not see the M128 menu.  To return from your

program to the program you stopped, use RST 0.  To revert back to the

Multiface normal operation, press NMI and BREAK (shift+space)

simultaneously.  This also cancels the code word RUN.

In standard mode M128 uses 8192-11144 as a buffer (8192-13496 once

you proceed to SAVE) and overwrites anything in there.  Using direct

jump, you have 8257-16338 available.

Clear - to clear the extra 64K RAM bank (in 128K mode only!).  You can

clear any time, but it is only useful with 48K programs in 128K mode.

However, you should save 48K programs in 48K mode anyway.  Also, for

obvious reasons, clearing the banks in 128K programs is not a good

idea.

The actual M128 has a software on/off switch, so that it could be made

completely invisible to programs until the NMI switch was pressed. This

software switch is not emulated.  The M128 could be turned off by

pressing 'O' in the main menu; this does not work on the emulator.  You

can turn off the M128 in the Select Hardware (F9) menu.

Using the extra 8K RAM - M128 has an 8K Rom containing its own

software, and an 8K RAM used as a buffer.  You can use the 8K RAM for

your own mcode routines or for data (but not for BASIC).  The RAM must

be paged in machine code to be accessed: use IN A,(191) to page in, and

IN A,(63) to page out.  The BASIC In and Out commands can not be used

here (they work, but result in a crash).  The 8K RAM overshadows the

Spectrum ROM and thus anything contained in the M128 RAM cannot make

any calls to the Spectrum ROM, as they both occupy the same area. The

M128 RAM routines should therefore be self-contained, independent of

the Spectrum ROM.

Poking infinite lives - To POKE, say 31000,0, first load the program as

usual.  When it's loaded, press NMI (F5) and select the tool by

pressing T.  When the toolkit menu appears, press SPACE and type 31000.

Once you type 5 digits the cursor automatically moves to the value

field (no need to press ENTER), so type 0 (value is 1-3 digits long)

and this time press ENTER.  Finally press Q to quit the toolkit and R

to return to the program.

The Multiface 128 was designed by Romantic Robot UK Ltd, 54 Deanscroft

Avenue, London NW9 8EN, tel. 081-200 8870.

2.13  Using the RS232 channel

This was the only Spectrum i/o channel that could be used in the early

versions of the emulator.  Using .TAP files instead of the RS232

channel is often easier, but sometimes using the RS232 channel can be

very useful too, for instance if you've got a null-modem lead that

connects a Spectrum with interface I to the PC you can use it to

transfer data and programs easily.  Furthermore, the RS232 channel is

the easiest way to let the emulator communicate with a PC printer.

Several things send their output to the channel designated as 'RS232

output channel'.  First of all the ordinary Interface I RS232 "B" and

"T" channels.  Secondly the 128K printer "P" channel.  Thirdly, ZX

Printer output is converted to a format appropriate for Epson or HP PCL

printers (works for most dot matrix, and most laser/inkjet printers

respectively) and also sent to this channel. Input from the 'RS232

input channel' is sent to the Interface I's "B" and "T" channels.

The Interface I RS232 port, the "B" or "T" channel, behave slightly

differently.  The first, binary, channel is the raw channel.  It will

let all data go through unchanged.  The "T" channel won't let all

control codes through and will expand any keyword; useful for LISTing a

program but otherwise annoying.

The Spectrum 128 has its own RS232 port; it is called the "P" channel.

Output to either the Interface I's or Spectrum 128's own RS232 port

will all be processed as 'RS232 output'.

The output to the RS232 channel can be routed to an LPT port, to a COM

port or to a file on disk.  Input can come from either a file or a COM

port.

If you want to use the RS232 channel for printing using LPRINT and

LLIST (shorthand for PRINT #3 and LIST #3), be sure to open that

channel for output to RS232; by default it sends its output to the ZX

Printer, which is not supported.  You can open the channel by typing

OPEN #3,"B" (or "T" for listings, or "P" on a Spectrum 128).

Input and output are buffered.  This is important to remember when

you're transferring files using the SAVE and LOAD *"b" commands of the

Interface I.  If the header is missed, for instance if you try to load

the wrong file type, re-sending the file will not directly work because

there will still be bytes in the buffer.  You have to clear the input

buffer before re-sending the file.  When inputting from a disk file,

the file pointer can be reset to point to the start of the file again

to re-read the header.

When inputting from or outputting to a disk file, the read or write

position is displayed as a byte-count.  An <EOF> sign will appear if an

input file is read completely through to the end.

The RS232 redirection options are in the Change Settings (F4) menu.

When using a COM port, make sure you have initialised it before

starting the emulator with the Dos MODE command, for instance

MODE com1:96,n,8,1

initialises COM1 to send and receive at 9600 baud, no parity, 8 data

bits and 1 stop bit, the default for the Interface I.

Here is how to transfer programs from a Spectrum to the PC using the

RS232 lead.  First, you need a null-modem lead.  I myself use the

following cable:

Spectrum                                  'AT'      'PC'

(9 pins)                                (9 pins)  (25 pins)

3 TxD   ������������������������� RxD     2         3

4 DSR   ������������������������� DTR     4        20

����� CTS     7         4

����� RTS     8         5

7 GND   ������������������������� GND     5         7

(so CTS and RTS have to be connected!)  This is not a full null-modem

lead; you can only send data from the Spectrum to a PC.  If you have an

Interface I, you can use the LOAD *"b" and SAVE *"b" commands on both

your real Spectrum and the emulator to transfer programs and data. It's

best to first type LOAD *"b" and then SAVE instead of the other way

around.  If you have problems with transferring data, try to lower the

baud rate to 4800 or 2400 baud; this sometimes helps.

If you don't have an Interface I, but do have a printer interface using

RS232 leads, then you can transfer data using LPRINT on the Spectrum

side, and INKEY$ #3 on the emulator side.  It needs a little bit of

programming.

2.14  On joysticks and mice

The emulator support several joysticks and the Microsoft mouse, to

control several Spectrum joystick interfaces, and the AMX Mouse

interface.  First, joysticks are discussed.

As was already said in the introduction, the emulated Spectrum joystick

(Cursor, Interface 2, Kempston or user defined) is controlled by the PC

cursor keys and 5/0/.  on the numeric keypad and TAB as fire keys.  The

emulated joystick can also be controlled by a mouse, or by a real

joystick, either analogue (PC standard) or digital.

The analogue joystick support is rather straightforward.  If you've got

one, it works - it couldn't be simpler.  The digital joystick support

is less obvious, since PC's don't support these.

To use digital joysticks, Ruud Zandbergen has made a device that uses

the two inputs of a normal analogue joystickinterface to connect a

digital joystick to a PC.  Here's the circuit diagram:

15 pins male  (pc)                           9 pins male (joystick)

1+9 <���������������������������������������������������> 7 (5V)

��         ��          ��           ��

4 x 1 k� � � � Watt  � �          � �           � �

���         ���          ���           ���

3   <������������> 4 (up) �            �             �

�            �             �

6   <������������������������> 3 (dwn) �             �

�             �

13  <�������������������������������������> 1 (rght) �

�

11  <���������������������������������������������������> 2 (lft)

��������������������Ŀ

2   <�������������Ĵ    47 � � Watt     ����������������> 6 (fire)

����������������������

4+5+14 <������������������������������������������������> 8 (0V)

4+5+14 means: connect pins 4, 5 and 14.  The same applies for pins 1 and

9.  Here's the list of ingredients:

1 x 9 pins D plug, male

1 x 15 pins D plug, male

4 x 1k� , � Watt resistors

1 x 47 �, � Watt resistor

piece of 7-wire flatcable

Everything can be fitted into the 15-pins plug. Make sure the resistors

don't touch the other blank connections! This interface can be used for

all usual digital joysticks, with or without auto fire (that is every

joystick that work with a Kempston joystick interface, or that work on

a Commodore 64/Amiga or Atari). The joysticks for the Spectrum +2/+3

will not work, however the pin layout is easy to change.

This joystick interface needs an analogue PC-joystick interface on

which you can connect TWO analogue joysticks (on one plug!).  Most

cards can do this, but some multi-I/O cards support only one joystick.

Check the documentation of your I/O card to see whether your

joystickinterface is suitable.  The soundblaster joystick interface

works fine.

A number of PC games will behave strangely when the digital joystick

interface is connected; they run very slow or crash.  When this

happens, remove the joystick interface (not only the joystick!).

With version 3, the Spectrum joystick (Cursor, Kempston, Sinclair 2 or

user defined) can not only be controlled by the cursor keys or a real

joystick, but also by a mouse.  Specify -km on the command line to have

the mouse control the joystick too.  (By default, this switch is

already in the Z80.INI file.)

Finally, the Microsoft mouse can control the AMX Mouse interface, as

supported for example by Art Studio.  Specify -ka on the command line

to select this option.  The mouse will not control the joystick

anymore.  Because of I/O address clashes, the AMX Mouse does not work

with SamRam or an M.G.T. interface enabled.  Also, since the AMX

interface must be initialized, and the state of the AMX interface is

not saved in the .Z80 snapshot file, it may be necessary to make the

snapshot before the point at which the Spectrum program initializes the

AMX interface.  The program Art Studio re-initializes the AMX interface

regularly, however, so with this program you don't have to worry about

it.  You still need to customize the program to use the AMX mouse of

course, so a snapshot of Art Studio will not automatically work; you

need to load the original from tape again, and go through the

customization procedure.

2.15  The utility ConvZ80

This program converts between various snapshot formats, and it can also

convert various tape file formats to .TAP files.  Currently it supports

.SNA format (used in various emulators, such as JPP, Peter McGavin's

emulator on the Amiga, and XZX), the .SP format of VGASPEC and

SPECTRUM, the .PRG format of SpecEm, and all old .Z80 file formats.

If conversion is to a .Z80 file, the old (v1.45) format will be used.

This is still supported by the emulator, and some other programs do not

support the new format.  Note that, because of this, ConvZ80 will not

convert .Z80 files containing snapshots of 128K or SamRam programs.

CONVZ80 recognizes what it should do by the extension of the files you

enter on the command line; to distinguish between VGASPEC's and

SPECTRUM's .SP formats you can use the switch -o.  If the extension

consists of digits only, it is taken to be a ZX tape file, and if it

contains non-digits and is none of .SP, .Z80, .SNA, .PRG or .TAP it is

regarded as a SpecEm tape file.

SpecEm can load .PRG snapshot files, but cannot save them.  However, it

emulates the Multiface I, which can save snapshots to tape.  SpecEm

will save these blocks as tape files to disk.  If you convert these to

a .TAP file (in the correct order!), you can load them into Z80 and

save the program as a .Z80 file.

2.16  Converting file formats - the utility CONVERT

This section is about the utility CONVERT, which can convert some of

the Spectrum's own format into each other, and also converts some of

the emulator's formats into others.  It is not about converting files

from other emulators; read section 2.15 if you want to know about that.

CONVERT was useful when the emulator could only communicate with

snapshot files and the RS232 link.  It has become less useful now, with

.TAP files and the possibility to load and save blocks directly into

and from Spectrum memory, but it still has some useful features.

It can read three types of input files: pure ASCII, pure bytes (for

instance a .SCR screen dump), and files produced by a SAVE *"b"

command.

Output is pure bytes, ASCII with either CR (Spectrum standard) or CR/LF

(PC standard) for line breaks, SAVE *"b" files containing a Basic or

code file, a .PCX or a .GIF file.

So what can you do? Main uses are adding LF (10 hex) bytes to a text

file produced by the Spectrum; converting a code block into a SAVE *"b"

to load it into the Spectrum using LOAD *"b" (and the reverse of

course: converting a SAVE *"b" file to pure bytes), and converting a

screen dump to .PCX or .GIF graphics files.

Less useful, but possible: LISTing a program (SAVE *"b" file) to

produce readable ASCII, and the reverse: converting an ASCII listing to

executable Basic again.

If you want to make a .PCX or a .GIF file, input should be a SAVE *"b"

file of a screen (length 6921 bytes exactly) or a bare .SCR screendump

(length 6912 bytes).  You can make screendumps by selecting the X-Extra

functions menu from the main menu.

2.17  The utilities Z802TAP, TAP2TAPE and TAP2VOC

The SamRam has built in it some snapshot software.  Using this software

you can save any 48K Spectrum program to tape or to a .TAP file, as is

explained in section 3.2 below.  But the SamRam software cannot handle

a 128K program.  The Multiface 128 can also write a snapshot to tape,

but in some cases it is still preferrable to use TAP2TAPE, since the

latter produces less and shorter blocks, and doesn't corrupt the screen

as much as the Multiface code does (on Spectrums without Multiface).

The utility that can convert a 128K snapshot (and 48K ones for that

matter) to a .TAP file is called Z802TAP.  The .TAP file includes a

basic loader, and a loading screen if you want.  Z802TAP compresses the

blocks it writes (using a better method than used in compressing .Z80

files) to save loading time.  If you don't want it to compress the

blocks, for instance when you want to take a look at the ram pages of

the Spectrum 128, specify -u when you run Z802TAP.  You can load the

converted program simply by executing

Z80 -ti tapefile

and typing LOAD "" (for a 48K program) or changing the hardware mode to

Spectrum 128 and choose 'Tape Loader' in the menu.

Of course you could also use Multiface 128 or the SamRam to convert a

snapshot to a .TAP file.

The program TAP2TAPE writes .TAP files back to tape.  The program

consists of a batch file TAP2TAPE.BAT, which executes the TAP2TAPE.Z80

file using the emulator.  The .TAP file is written to tape exactly as

it is, so that if a block contains a tape error, it won't load

correctly from tape either.  If the entire .TAP file has been saved the

emulator will start loading from tape.  At that point, press space once

to return to DOS.

The TAP2VOC program converts the tape block in a .TAP file to the

corresponding bips and clicks.  The resulting .VOC file will be rather

large.  It can be loaded back into the emulator, which is rather silly

since using the original .TAP file is better in all respects, or you

can write the program to tape using a sample player.

2.18  The utility OUT2VOC - and how to make .OUT files

The emulator can log OUTs to any I/O port.  These OUTs are logged in a

.OUT file, in which is stored what was written to which port at what

time.  These log files are used for recording sounds the emulated

Spectrum produces.

The OUTs to port FE, which controls the internal beeper, and those to

ports BFFD and FFFD, which control the soundchip of the Spectrum 128,

are translated into a sample file by OUT2VOC.

There are a number of command line switches.  The sample frequency can

be chosen by specifying "-f frequency" on the command line.  By default

a sampling frequency of 10 kHz is used.

Ordinarily the OUT2VOC program filters out all frequencies above half

the sampling frequency, so that no aliasing occurs when the signal is

stored as a sequence of samples in a .VOC sample file.  For sample

files containing tape data, these aliasing effects are not important.

For these sample files you can use -d to produce 'digital' sample

files, containing only 'high' and 'low' sample values, and nothing in

between.  These sample files can be compressed much better.

By default, the OUT2VOC program listens to the EAR output only.

Specify -m to have it listen to the MIC output (only), and -a to have

it listen to the AY-3-8912 soundchip of the Spectrum 128.  If you

specify for example -e -a, the program will listen to both

corresponding channels.

Usually, not all registers of the AY soundchip are updated continually.

Some registers are initialised and left at that value.  So it may be

necessary to supply the initial values of the soundchip registers.  Do

this by specifying '-i file', where 'file' is the snapshot taken just

before you started recording the OUTs.

Some Spectrum programs use extremely many OUTs to produce sounds.

Fairlight for example OUTs about 70000 times a second, thereby indeed

producing one of the finest music I've every heard coming out of an

ordinary Spectrum.  The OUT2VOC program takes quite some time to

compute the .VOC file from the raw .OUT output.  Use -q for a quick

and little less precise conversion.  The difference seems to be barely

audible.  This switch has no effect when converting AY soundchip music.

If you specify -s, silences of longer than 1 second will be truncated

to 1 second.

Finally, use -r to produce a raw sample file, without any header or

length info.

To record a sample, go to the Extra Functions menu (F10, X), and select

O.  Select the ports you want to log (by default only FE is logged) and

enter a name for the log file.  OUTs will be logged until you specify

an illegal name, or press ENTER on an empty line here.

To save a block of data to a .OUT file, simply type the right SAVE

command or use any (turbo-save) program to save it to tape.  Note that

.OUT files tend to grow fast; one bit on the tape corresponds to two

OUTs and therefore two 5-byte entries in the .OUT file; a 48K file will

produce a .OUT file of at least 4 Megabytes.

If you give a 16 bit address for an I/O port, only OUTs to this address

will be logged.  If you give an address smaller than 100 hex, all

addresses whose low byte equal this number are logged.  Note that it

is not possible to log all OUTs to, say, an even port.  Although the

MIC and EAR ports respond to all even port addresses, virtually no

program uses a port different from FE, so this will be no problem in

practice.

Note that whereas the 'official' addresses of the register and

value-port of the AY soundchip are FFFD and BFFD respectively, some

programs use other ports that work too.  One particular program uses

BEFD, heaven knows why.  For these programs, specify FD (or 00FD) as

OUT port to log; this will ensure that all OUTs to any address with low

byte FD is logged (which, alas, includes the much-used port 7FFD).

A recent discussion on comp.sys.sinclair revealed that the AY

conversion of OUT2VOC leaves much to be desired.  It is on my list of

things-to-do.

2.19  The utilities READVOC and READSB

These programs can read samples from the LPT port or the SoundBlaster

and store them in .VOC files, for later play-back to the emulator.

This is useful when you use the SoundBlaster as input device, as direct

loading is not very reliable then because Z80 does not filter its input

appropriately; you can store level data of games on hard-disk and load

them without having to play the actual tape again; thirdly, this allows

you to try to load the program with different emulator settings easily.

READVOC uses polling to read the sample.  This method does not allow

for very high sample frequencies (even 20 kHz is too much sometimes),

and does not work reliably in multi-tasking environments (DOS boxes of

Windows (95) for example).  It is however the only way to take samples

from the LPT interface, and works well under DOS.

READSB has been written for taking samples from the SoundBlaster.  It

uses DMA to read the sample, and applies a digital filtering algorithm

for reliable loading.  It offers a number of command-line switches.

Digital filtering is highly recommended: for programs that use the

ordinary tape format, -n is appropriate; for turbo-save formats, use

-d.  If neither of -d and -n are used, ordinary analogue sample files

are produced, which will very often not load successfully when played

back to Z80.  For low sample rates, say 10 kHz, it is a good thing to

use two-times oversampling (switch -2).

When READSB is started, it displays a signal level bar.  Set the volume

so that a reasonable input level is reached; do not increase the volume

too much above 50%.  Press any key to start sampling, and then 'p' to

pause and display the signal level bar again, or any other key to save

the sample and quit.

READSB uses 0220 as default base SoundBlaster I/O address, and IRQ 7 as

default interrupt level.  Be sure to set these to their appropriate

values if your card is not set to these defaults.

Both READVOC and READSB produce very large files.  Be sure to have

several megabytes of free disk space available.  After sampling, these

sample files can be compressed using ordinary compression programs to

very small sizes.

2.20  Quick overview of command line switches, and features

In this section I will briefly explain all command line switches.  It

is useful to at least once read this section carefully; there are a few

small handy features that will otherwise be readily overlooked. Starred

switches are by default put in the Z80.INI file.

-h        Hercules graphics

-c        CGA graphics

-e        EGA graphics

-v        VGA graphics

Usually the emulator will by itself correctly determine which mode to

use.  VGA mode is by far the best: it is the only mode in which all

colours are correct (including the bright hues), and it is also the

fastest mode.  Versions prior to v3.04 also had Plantronics and Extended

Hercules as possible video modes; these are removed to save memory.

-xv       400-line VGA (for some Tridents)

-xb       Black-and-white VGA

The black-and-white switch is useful on black-and-white monitors;

sometimes those only display one component of the RGB signal instead of

a weighted average, so that some colours become indistinguishable grey

tones.

Some Trident video adapters, as well as some others, have a bug in

their video BIOS: they don't want to be put in 200-line mode.  To get

around this, when -xv is specified 400-line mode is used with a double

amount of lines.

-n        Emulate Spectrum without Interface I

-s        Emulate Spectrum with SamRam

-1[28]    Emulate Spectrum 128 (can be used with -n)

-xk       Emulate Multiface 128 interface  (can't be used with SamRam)

-9        Default to M.G.T. interface instead of Interface I

-0d       M.G.T. = DISCiPLE ROM (default)

-0p       M.G.T. = Plus D

These switches select the default start-up hardware configuration of

the emulator.  This can all be changed in the Change Hardware menu

under function key F9.

*  -znnn.n   Emulate the Spectrum at speed nnn.n%

The most sensible setting for this is -z100.  By default this is done

in the Z80.INI file.

-xx       Hi-resolution colour emulation

This selects hi-res colour emulation by default, so that border effects

and hi-res colour effects are visible.  This setting can be changed in

the F4 menu.

-r        Emulate the R register

This is usually necessary in turbo loaders and otherwise protected

programs, since they almost invariably use the R register to decode

things.  The R register has to be updated every instruction, so that

enabling the emulation of it reduces the emulation speed considerably,

by some 30%.  Don't do it unless it proves to be necessary.  I don't

know of any program that uses it on the run.  If the R register is not

emulated, the R register acts as a random generator.

*  -l        Better (but slower) LDIR/LDDR emulation

Normally a 8086 MOVSB is used to emulate an LDIR or LDDR.  This is a

fast instruction.  Programs that use them a lot, most notably Jetset

Willy and Manic Miner, run very fast.  On slow computers these programs

are the only ones that are really playable.  On fast computers they run

much too fast, so it is a wise thing to use -l on fast computers.

Secondly, without -l the LDIR instruction is not emulated correctly

when it overwrites itself; with -l it is.

-yl, -yh  Low, high video synchronization mode

These switches control the phase between the 50 Hz interrupt and the

screen refresh.  Some programs need a different setting in order to

remove flickering of moving characters.  Enabling hi-resolution colour

emulation also eliminates these problems.

-2        Emulate an Issue-2 Spectrum

Make the EAR line 1 when there's no signal, instead of 0 as it is on

Issue 3 Spectrum's.  Some old programs need it (Spinads for example).

-dN       Set frame interrupt frequency to 100/N Hz

There used to be an option -d which set the frame interrupt frequency

to 100 Hz; therefore now -d is equivalent to -d1.  -d2 is the default

setting, 50 Hz.  On slow computers, -d helps to get a better keyboard

response.  On the other hand, setting for instance -z350 -d7 makes Z80

emulate a Spectrum 3.5 times as fast while maintaining normal keyboard

response.  At run time, the frame interrupt rate may be toggled between

50 Hz and the value set by this switch.  If you set -dN twice, then the

emulator starts off with a 50 Hz frame rate, with alternate rate set to

100/N Hz.

-aN       Redirect Spectrum RS232 output to LPTn

-oN       Redirect Spectrum RS232 output to COMn

-uF       Redirect file F to Spectrum RS232 input

-iN       Redirect COMn to Spectrum RS232 input

-wF       Redirect Spectrum RS232 output to file F

All output to, and reading from, the Interface I's RS232 channel

(channels "b" and "t"), the 128K printer channel (channel "p") will be

sent to, respectively read from, the appropriate device.  Furthermore,

output to the ZX Printer will be converted in a format suited for Epson

matrix printers or HP Laserjet printers and also be sent to the 'RS232

output' device or file.

-g        No sound

-xa       Do NOT use AdLib for 128 sound

-xc       Use SoundBlaster CMS chips for noise

-xi       Do NOT use internal PC speaker for '128 sound

If no AdLib compatible card is detected, 128K sound output will be

sent, as far as is possible, to the internal PC speaker.  Use -xa to

hear 128K sound through the PC speaker when there is an AdLib card

present.

Without CMS chips, the 128K soundchip's noise cannot be faithfully

reproduced.  If you have CMS chips installed, use -xc to use them.  If

the SoundBlaster is on a different base address than 0220, use -xq to

select it.

*  -km       Use MS Mouse to control Spectrum joystick

-ka       Emulate Spectrum AMX mouse interface controlled by MS Mouse

-kz       Use digital Zandbergen joystick

-kk       Do not look for analogue or Zandbergen joystick

-k1       Do not use 2nd analogue joystick as Sinclair 1 (1,2,3,4,5)

-k2       Use 2nd joystick as Sinclair 1 always

The emulated joystick (Cursor, Kempston, Sinclair 2 or user defined) is

controlled by the arrow keys and TAB, 5, INS (0) on the PC keyboard,

and optionally also by a digital joystick, and analogue IBM joystick or

a mouse.  Use -kk if the emulator erroneously detects either joystick.

If -ka is specified, the PC mouse controls the emulated AMX mouse

interface.  Because of I/O address clashes, this only works with SamRam

and the M.G.T. interfaces disabled.  The AMX mouse is supported by e.g.

Art Studio.

A second analogue joystick will, when present, always be attached to

keys 1 to 5, the Sinclair 1 joystick keys.  If neither -k1 nor -k2 is

given, the emulator will look if the second joystick is in an

acceptable neutral position (i.e. is attached) and if so will use it.

This will usually work.  If it incorrectly recognizes the joystick use

-k1.  If it doesn't recognize a joystick while it is connected, use

-k2.

-jc       Emulate Cursor joystick (default)

-jk       Emulate Kempston joystick

-j2       Emulate Sinclair joystick 2 (keys 6,7,8,9 and 0)

-ju<lf><rt><up><dn><fire>  User defined joystick

Also quite clear I think.  For instance, -juipqzm makes the joystick

control the i,p,q,z and m keys, for instance to play those lovely

Horace games.  Special characters are [ for shift, ] for symbol shift,

/ for enter and \ for space.

*  -xs       Default to shifted cursor keys (NumLock) in '128 mode

By default, the PC arrow keys control the cursor keys 5,6,7,8 and 0 of

the Spectrum.  The Spectrum 128 had its arrow keys press the cursor

keys plus shift, and the menu bar of the reset screen of the '128 only

responds to shift+cursor key.

-bN       Use LPTn for tape I/O

-xo       Use SoundBlaster's A/D Converter for tape input

-xq adr   Set base IO address for SoundBlaster (default 0220)

Either the LPT tape interface of the SoundBlaster is used for tape

input.  Only the LPT tape interface can be used for tape (MIC) output.

-tv F     Play .VOC file F to 'ear' input

-m N F    Insert cartridge file F{.MDR} into microdrive N

-xm D     Select default directory D for .MDR files

-td D     Select directory D for Multiple .TAP File mode

-ts D     Specify default directory for Single .TAP File mode

-ti F     Use F{.TAP} as tape input

-to F     Use F{.TAP} as tape output (if exists, append)

-tm       Mirror tape input to disk (Do also specify output file!)

-te       Mirror tape input to disk in EXACT mode

-tw       Don't wrap load pointer of tapefile at end

-tp       Pause after loading .TAP block

All quite clear I think.  If -tv is used, the VOC file starts playing

immediately, so be sure there's an appropriate snapshot running to do

something with the signal.  -tw makes the emulator load from physical

tape when a .TAP file has been read to the end, instead of starting

again from the beginning.  It is used in Tap2Tape.

-xl N     Select LPTn for ZX-Printer output

-xj       'Off-line=wait' instead of 'Off-line=No ZX Printer'

-0e       Epson printer (for ZX PRINTER & M.G.T.)

-0h       HP PCL printer (for ZX PRINTER & M.G.T.)

-0f N     Issue a Form Feed every N ZX Printer lines when printing

to a HP PCL compatible printer

Output to the ZX Printer will be converted into graphic data in Epson

(most matrix printers) or HP PCL (most inkjet and laser printers)

format.  Use -0e or -0h to select which format to use; default is Epson

format (defaulted in Z80.INI).  Also, this selects the default DISCiPLE

ROM to be used; the two ROMs have different screen dump routines

pre-loaded.  If you're using a Laser printer, use -0f 103 to have Z80

eject the page once in a while.  103 lines approximate one sheet A4.

Select the LPT port to print the data to with -xl; default is LPT1.

If the printer to which ZX Printer output is sent is absent, busy or

off-line, the emulator tells the Spectrum program that the ZX Printer

is not attached.  If you want the Spectrum program to wait in case of

an off-line or busy printer, then put -xj in the Z80.INI file.  This

switch is not defaulted because printers sometimes say they're merely

busy or off-line when in fact they are not there at all.  Besides, some

Spectrum programs do a COPY without asking, and with this switch the

program would lock up (as versions 3.00 to 3.02 did; in v3.03 and above

you can BREAK into such a loop).

If a printer becomes off-line or unavailable during printing, Z80 will

try to finish its job anyhow, and the program may appear to hang.

Pressing the space bar will make Z80 break the printing loop, and

signal to the Spectrum program that the ZX Printer does not exist.

This, in turn, will cause the ROM printer routines to just skip

printing.  Z80 will periodically check the printer status, and when it

becomes available again will make the ZX Printer appear again.

-0i       Disable Disciple interrupt

Normally the Disciple and Plus D have their own interrupt routine being

executed 50 times a second.  Because this involves many page swaps,

this is quite slow on the emulator.  Since the interrupt is not really

necessary anyway, it is reduced to once every two seconds.  On some

computers this may still be too much, in which case -0i completely

disables the Disciple and Plus D interrupts.  Do not try this in

Unidos!  (which is not emulated [yet].)

-0sND     Set Disciple/+D drive N to D:

By default, the Disciple/+D drive 1 controls drive A: while 2 controls

B:.  If, for example, your B: drive is your only 3.5'' drive, you might

want to map both 1 and 2 to B: which is accomplished by putting -0s1b

in the Z80.INI file.

-xp D     Select default directory D for .Z80 snapshot files

Clear.

-xz       Assume AZERTY keyboard layout

Assumes the Belgian and French AZERTY keyboard layout.

-xr F     Use file F as standard Spectrum rom

The ROM will be used in 48K modes only, and must be exactly 16K long.

-xt       Use as little memory as possible with full functionality

-xu       Use as little memory as possible; disable HRC emulation

With -xt, all unnecessary buffers used for efficiency purposes only are

reduced to minimum size.  Saves 47K, but can make all the difference

sometimes.  With -xu, 83K is saved, but Hi Resolution Colour emulation

won't work anymore.

-tx       Do not display tape info window when loading or saving

-00       Do not show intro screen, and don't pause at startup

Some people seem to really hate either, or even both.  Don't know why.

For better annoyance, -00 only works in the registered version.  Hehe.

-xe       Do not use expanded memory

Specifying this switch will result in the emulator using 240K more base

memory (making a grand total of 564K).  Furthermore, all page swapping

will be much slower, so especially the 128K emulation gets verrry slow.

-xg       Log all outs to FE in OUT log file

-xy       Dump trace of program in .OUT file

By default, OUTs to port #FE that do not change the state of either MIC

or EAR are not saved to the log file, to save disk space.  If, for some

reason, you want all OUTs to be logged, -xg will do the trick.  The -xy

option can be used for producing a somewhat crude trace; see section

5.10 on the .OUT format for more details.

-0m F     Build map of opcode locations in file F

When this option is specified, Z80 keeps a map of the 65536 address

locations in memory, and tags the address of each instruction that is

emulated.  When the emulator is shut down, this map is written to disk

as a 8192 byte file.  See section 5.10 of TECHINFO.DOC for an

explanation of the file format, and remarks on how to use this feature.

-xw       Run in Windows compatibility mode

In this mode, the timer is not re-programmed.  The emulator counts T

states to see when an interrupt has to be generated (so emulation will

be slower).  In a Windows (3.1 or 95) DOS box, using -xw will turn

Windows compatibility mode off (as Z80 is aware of Windows and has

turned this switch on itself in that case) and the emulator will

usually hang.  There is probably never a need to use this switch.

-kxABCD   Press key #CD for scancode #AB.

This can be used to re-map the keyboard, for instance if your C key is

broken, you can re-map it to F11, or any other key for that matter.

Combinations are hard-coded, so that a key pressing , also presses <

when shifted.  The scan code is between #01 (ESC) and #58 (F12); a full

list can be found in many places.  The format of the key byte is

explained in section 2.3.

-xf       Switch full 16K of Interface I ROM

Useless.

2.21  Miscellaneous remarks

1. Problems at the Rom/Ram boundary

There are a few Spectrum programs that have an odd stack pointer, and

run over the ram/rom boundary, for instance Deep Strike, and Elite.

This crashed version 1.45 of the emulator; the problem was circumvented

somewhat in version 2.01, and has been removed in version 3: in most

cases, a word read from or written to FFFF will be read or written as

two bytes.  You can check this by typing CLEAR 65535: POKE 65535,0:

RETURN in BASIC; this will lock up version 1.45 of the emulator, it

will lock up version 2.01 if the 80386 is in virtual mode, and it will

only crash the emulated Spectrum (as it should) on version 3.

The check on FFFF is not done at every potentially problematic op-code,

because this would make the emulator noticeably slower.  It is

therefore still possible to 'hang' the emulator.  Not going to tell you

how, though.  All existing programs seem to work okay now.

2. Critical timings

A few programs (the only examples known to me are Fireman and Thing,

but there are more) are quite sensitive to the relative actual

execution speed of emulated Z80 instructions, and crash if it isn't

right.  They rely on this-or-that amount of instructions to be executed

between interrupts.  If you slow down the emulator, these program will

run fine, because then individual instructions are more carefully

timed.  When running in Windows compatibility mode, loading .VOC files

or when the Hi-Res Colour emulation is switched on, these problems

disappear completely as the emulator is then counting off the exact

number of T states to elapse between interrupts.

3. Shifted cursor keys

It may be annoying to have to press Num-Lock every time you use the

Spectrum 128 (because otherwise you'll have to use Shift with the

cursor keys to move the menu bar).  To make the emulator press shift by

default every time you use the PC cursor keys in '128 mode, use the

switch -xs. If you press Num-Lock now (in '128 mode), the shift-key

won't be pressed.  The 48K modes are not affected by this switch.

4. Running non-standard ROMs

To run the emulator with a different rom than the standard one, you can

specify a rom image file on the command line.  Use the switch -xr file,

where 'file' is the name of the image file.  This file should be

exactly 16384 bytes long.  It will of course not be used in Spectrum

128 or SamRam mode.

The emulator 'ZX' by Rindt and Bruckner comes with several roms, stored

in their tape format.  You can convert these files to .TAP files, and

then load them in the normal way (in RAM), and then save the 16K image

from RAM directly.  You can also extract it from the files directly by

using the DOS debug utility:

C:\>debug rom.000       (or other rom file (of 16406 bytes))

-m 115 L 4000,100       (move the rom down, overwrite header)

-rcx                    (new length of exactly 16K bytes)

CX 4016

4000

-n rom000.bin           (or some other name)

-w                      (write it)

Writing 04000 bytes

-q                      (and quit)

5. Printing with DISCiPLE and Plus D

If, emulating a DISCiPLE/+D and by entering POKE @11,0 the parallel

printer output has been selected, going back to a hardware mode without

M.G.T. interface causes problems with printing to the ZX Printer, since

the output addresses of the "p" channel have been changed by the M.G.T.

ROM.  Use the "b" channel of the Interface I instead.  You can reset

the channel to the ZX Printer by entering POKE @11,1 while still

emulating the M.G.T. interface.  Note however that the printer OUT port

#FB of the ZX Printer is used by the Disciple printer interface, so

that using the ordinary ZX Printer code of the ROM results in rubbish

coming out of the printer.  The +D can use the Centronics interface and

the old ZX Printer at the same time.

Although by default channel #3 prints to the ZX Printer, in all

hardware modes, if a DISCiPLE/+D is emulated output to the ZX Printer

is ignored, as these interfaces use the ZX Printer I/O ports for their

own purposes.

6. Breakpoints

A Breakpoint may be set in the Extra Functions menu, option B.  If it

is set, the emulator will at the beginning of each instruction check

whether the program counter is equal to the breakpoint address.  If

this is the case, it notifies the user, and continues to the main menu.

You could now save the snapshot and inspect register values using

Z80Dump, for example.  You could change hardware mode to SamRam, and

generate an NMI from the menu (using Extra Functions, N) to inspect the

code around the breakpoint.

Some fine points.  If a breakpoint is hit, the instruction at the

breakpoint address is not yet executed.  It will be upon returning.  If

you generate an NMI, and subsequently return from it, the first

instruction that will be executed after the NMI is the instruction at

the breakpoint; this will thus make the emulator go to the main menu

once again.  Press ESC to continue emulating.

For technical reasons, when counting T states, the time taken by the

instruction directly preceding (in execution) the one at the breakpoint

address is not taken into account.  When the emulator hits a

breakpoint, the current frame will be a few T states longer.  Not that

you'll notice, probably.  The R register is counted through properly

(if it's emulated, that is.)

If the breakpoint address happens to point to an operand instead of an

opcode, the emulator will not respond.

7.  Emulation of floating bus

Reading from e.g. port #FE accesses the ULA bus which is used to read

the screen memory.  While the ULA is generating the border or

horizontal or vertical retrace signals, this will result in #FF;

otherwise, the value returned is the ATTR or screen byte the ULA is

currently reading.  Some programs use this to check whether the ULA is

in vertical retrace (Arkanoid), some use it to actually wait for

specific bytes to pass, which signals that a certain part of the screen

is being generated (Ricochet).  Only in Hi Res Color mode will the

emulation of this effect be reasonably accurate to get Ricochet running

correctly.  Arkanoid runs reasonably well, but due to the way it is

implemented it produces 'ticks' in the speaker when it is not run in

HRC mode.

3.  THE SAMRAM

3.1  Basic extensions

The SamRam is a hardware device Johan and I built for our Spectrums. It

consists of a 32K static RAM chip which contains a modified copy of the

normal Basic ROM and a number of other useful routines, like a monitor

and snapshot software.  You can compare it to a Multiface I interface,

but it's more versatile.  Another useful feature was a simple hardware

switch which allowed use of the shadow 32K Ram, present at 8000-FFFF in

most Spectrums, but hardly ever actually used.

For more details on the low-level hardware features of the SamRam read

chapter 5.  In this chapter I'll explain the software features of the

SamRam software, somewhat bombastically called the 'SamRam 32 Software

System' or the 'Sam Operating System'.  By the way, all similarity

between existing computers is in fact purely coincidental and has in no

way been intended.  Really!

The SamRam offers a few new Basic commands, and a lot of useful

routines that are activated by an NMI, i.e.  by pressing F5.  First

I'll discuss the Basic extension.

Select the SamRam by starting the emulator with the -s switch, or by

selecting it from the F9 menu.  Normal Basic functions as usual; the

character set is different from the original one.  There are four new

commands: *RS, *MOVE, *SAVE and *SPECTRUM, and two new functions, DEC

and HEX, which have replaced ASN and ACS.  DEC takes a string argument

containing a hexadecimal number, and returns the decimal value of it.

HEX is the inverse of the DEC function, and yields a four-character

string.

*RS sends its arguments directly to the RS232 channel.  You don't have

to open a "b" or "t" channel first.  You're right, it's of limited use.

Example: *RS 13,10

*MOVE is useful: it moves a block of memory to another place.  Example:

*MOVE 50000,16384,6912 moves a screen-sized block from 50000 to the

start of the screen memory.

*SAVE works like *MOVE, except that it activates the shadow SamRam ROM

before moving.  I used this command to update the shadow ROM, but on

the emulator you can use it to move the shadow ROM to a convenient

place in Ram where you can take a look at it, for instance by executing

*SAVE 0,32768,16384.

*SPECTRUM resets the SamRam Spectrum to a normal one.  You lose all

data in memory.  By resetting the emulator by pressing ALT-F5, the

SamRam is activated again.  Not very useful either.

Then there's the Ramdisk, which is, like the Spectrum 128 ramdisk,

accessed via the SAVE!, LOAD!, CAT!, ERASE! and FORMAT!.  The syntax is

straightforward.  FORMAT! and CAT! need no parameters; ERASE! only

needs a name.  If a file is not found, the SamRam will respond with a

5-End of File error.  The Ramdisk has a capacity of 25K.

3.2  The NMI software

Select the SamRam (F9-3), and press F5.  A menu with eight icons pops

up.  You can select each icon by moving the arrow to it (using the

cursor keys or the Kempston joystick), and pressing '0' or fire.  The

icons can also be selected by pressing the appropriate letter key.

The eight icons are two arrows with N and E within them, a magnifying

glass with the letters 'mc' in it (activated by pressing D), two

screens (identified by 1 and 2), a printer (P), a cassette (S) and a

box saying 'overig'.  The 'D' activates the monitor or disassembler;

read section 3.3 for information on this program.

Pressing N or E returns you to the Spectrum.  If you pressed N, the

normal Spectrum rom will be selected when the NMI software returns; if

you press E, the Rom with the Basic extensions will be selected.  Some

games may crash if they see a different rom than the standard Spectrum

one.

Pressing 1 selects the tiny screen editor.  You can move a '+' shaped

cursor about the screen using the cursor keys.  The following commands

are available:

H: Get the current ATTR colour from the screen at the cursor's

current position, and store it in memory.  This colour will be

used by the next command:

Z: Put the colour on the screen

G: Get a character from the screen

P: Put the character on the screen

R: Remove all screen data that is invisible by the ATTR colour

L: Take a look at the bitmap below the ATTR colour codes

T: Return to the main menu.  You can also return by pressing

EDIT, or ESC in the emulator.

B: Change border colour

V: Clear the whole screen

If you press 0, you can edit the current 8x8 character block at pixel

level.  Again you control the cursor with the cursor keys.  Now 0

toggles a pixel.  In this mode there are two commands: C clears the

whole block, and I inverts it.  Pressing EDIT (ESC) returns you to the

big screen again.

The SamRam has two screen buffers.  Buffer 1 is used to hold the screen

which was visible when you pressed NMI, to be able to restore it when

returning.  This is the screen you edit with '1'.  The second screen

buffer can be used to hold a screen for some time; it is not touched by

the NMI software directly, and will not even be destroyed by a Reset.

If you press '2', a menu appears with four Dutch entries:

1: Scherm 1 opslaan        (Store screen 1 into buffer 2)

2: Scherm 2 veranderen     (Edit screen 2)

3: Schermen verwisselen    (Swap screens)

4: Scherm 2 weghalen       (Remove screen 2)

These four functions are rather obvious, I believe.

Pressing 'P' pops up the printer menu.  The screendump program is

written specifically for my printer, a Star SG-10.  It will probably

work on some other printers, but not on most.  The output is sent to

the RS232 channel, so you have to redirect it to an LPT output.

Skipping the most interesting, 'S', for a moment, let's first discuss

the final menu, 'O' for 'Overig', Dutch for miscellaneous.  There are

five menu options, of which three are not useful.  The first gives a

directory of the cartridge currently in Microdrive 1.  The last, 'E',

returns you to Basic if this is anywhere possible: it resets some

crucial system variables and generates a Break into Program.  You can

use this for instance to break in a BEEP, or crack a not-so-very-well-

protected program.  The three other options select normal or speed-

save, and store the current setting in CMOS Ram.  Speed-save won't work

properly on the emulator, because the speed-save routine toggles the

upper 32K ram bank regularly, and this takes too much time on the

emulator.  The setting is not important if you use the internal save

routine (which will be used by default, unless you select Real Mode).

Finally, the 'S' option.  This option allows you to save a snapshot to

tape or microdrive.  I used it a lot on my real Spectrum, and it works

just as well on the emulator.  It is very useful is you want to load a

.Z80 program back into a real Spectrum again.  There are three

'switches' you can toggle.  The active choice is indicated by a bright

green box, inactive boxes are non-bright.  You have to use EGA or VGA

to be able to see it...  The first switch lets you select whether the

SamRam rom should be active if the program loads or not.  This is only

meaningful is you load it back in a SamRam again.  Usually I want the

SamRam rom to be active because I like the character set better.  The

second switch indicates whether the SamRam should save a 'loading

screen', which it takes from screen buffer 2.  If screen buffer 2

contains a screen, this switch will by default be on.  Finally, the

last switch lets you select the output media, tape or cartridge.

If the program is loaded back into the SamRam, the only bytes that have

been corrupted are four bytes down on the stack; this will virtually

never be any problem.  If the program is loaded back to a normal

Spectrum, these four bytes will also be corrupted, and the bottom two

pixel lines of the screen will be filled with data.  (This is

considerably less than any other snapshotter I've seen: for instance

the Multiface I uses more than 35% of the screen!)

The Microdrive BASIC loader needs code in the SamRam rom to start the

program (the RANDOMIZE USR 43 calls it).  It won't be very difficult to

write a standard BASIC loader that doesn't need this code, but I don't

think many people desperately need it...  Anyway, using the Multiface

128 you can write a compressed snapshot to cartridge which doesn't need

the Multiface.

3.3  The built-in monitor

This is a really very convenient part of the emulator, and I use it a

lot.  It is very MONS-like in its commands and visual appearance.  It

cannot single-step however, but on the positive side it has some

features MONS hasn't.  It is a part of the SamRam, and cannot therefore

be used with Spectrum 128 programs.  If you want to take a look at a

Spectrum 128 program, press F10, then change the hardware to SamRam

without resetting, and finally generate an NMI in the Extra Functions

menu.  You won't probably be able to continue to run the program, but

at least you're able to see what it was doing.

Press F5 for NMI, and D to enter the monitor/disassembler.  The first

eight lines are the first eight instructions, starting at the Memory

Pointer, from here on abbreviated by MP.  At first, MP is zero.  The

disassembler knows all official instructions, and the SLL instruction.

If another inofficial instruction (i.e.  starting with DD, FD or ED) is

encountered, the first byte is displayed on a blank line.  The four

lines below these display the value of PC and SP, the first nine words

on the stack (including AF and the program counter, which have been

pushed during NMI), and three MP-memories.  These can be used for

temporary storage of the MP, for instance when you take a look at the

body of a CALL, and want to return to the main procedure later.

The bottom part of the screen displays 24 bytes around the memory

pointer.

Commands are one letter long; no ENTER needs to be given.  If one or

more operands are needed, a colon will appear.  By default the monitor

accepts hexadecimal input.  A leading $ denotes that the number is to

be regarded as decimal.  If you give the # command, the default will

toggle to decimal, and you need to explicitly put a # in front of a

number which is to be interpreted as a hex number.  Also, after the #

command all addresses on screen will be decimal.  A single character

preceded by the " symbol evaluates to its ASCII code, and the single

character M will evaluate to the current value of the memory pointer.

The monitor commands:

Q: Decrease the memory pointer by one.  You effectively shift one

byte up.

A: Increase the memory pointer, shifting one byte down.

ENTER: Shift one instruction down: the memory pointer is

increased by the length of first instruction displayed on

screen.

M: Change the value of the memory pointer.  For instance, M:M

won't change it.

P: Put.  The word operand supplied will be stored in the first MP

memory, and the others will shift on place to the right.

Usually, you'll want to store the memory pointer by P:M

G: Get.  Typing G:1, G:2 or G:3 moves the value of one of the MP

memories to the MP.

B: Byte.  This command needs a byte operand; it will be poked

into memory, and the memory pointer will move one up.

I: Insert.  The same as B, except that you can poke more than one

byte.  It continues to ask for bytes to poke until you type

Enter on a blank line.

#: Toggles the default number base between hexadecimal and

decimal.

F: Find.  You can enter up to ten bytes, which will be searched

through memory.  Searching will stop at address 0, because

since the search string is stored in shadow Ram, searching

would otherwise not always terminate.  Typing Enter on a blank

line starts the search.  Byte operands are entered as usual,

but:

- If a number bigger than 256 decimal is entered, it is

treated as a word in the standard LSB/MSB format.  So, 1234

will search for 34,12 hex in that order.  Note that 0012

will search for 12, not 12,00.

- A line starting with " decodes into the string of characters

(up to ten) behind it.  Normally this would only be the

first character.  So instead of typing "M "Y "N "A "M "E

(space=enter here) you type "MYNAME.  Note that any

terminating " will also be searched for!

- An x is treated as a wildcard.  So if you search for CD x 80

any call to a subroutine in the block 8000-80FF is a hit.

If you search for x 8000, you'll see every one-byte

instruction that has the address 8000 as operand.

N: Continues the search started by F from the current MP.

$: Displays one page of disassembly on screen.  In this mode,

the following commands are possible:

$: Back to the main screen

7: [Shift 7 also works, cursor up]: Go to the previous page.

The monitor stores the addresses of the previous eight

pages only.

Q: Go back one byte (decrease MP by one)

A: Go one byte forward (increase MP by one)

Z: Dump this screen to the printer, in ASCII format.  Redirect

the RS232 output to a file, and run CONVERT on it to convert

the CR's into CR/LF's before printing (or tell your printer

to do the conversion).

Every other key displays the next page of disassembly.

K: List.  The same mode as with $ is entered, but instead of a

disassembly the bytes with their ASCII characters are

displayed.  Useful to look for text.

C: Clear.  Fills blocks of memory with a specified value.  The

monitor prompts with 'First', 'Last' and 'With'.  The 'Last'

address is inclusive!

D: Dump.  Prompts with 'First' and 'Last', and dumps a

disassembly of the block between these addresses to the

printer.  See remark at $-Z.  The 'Last' address is again

inclusive.

R: Registers.  If you press Enter after R, an overview of the

registers contents is displayed.  If you type one of A,B,C,D,

E,H,L,A',B',C',D',E',H',L',I,R,AF,BC,DE,HL,AF',BC',DE',HL',

IX,IY,SP or PC, you can change the value of it.  Changing the

value of SP also changes the PC and AF values by the way.  You

cannot change the Interrupt mode or IFF.

V: Verplaats.  (Move).  Prompts with 'From', 'To' and 'Length'.

Obvious.

S: Save.  Enter the start of the block you wish to save first.

The monitor then prompts with 'Length'.  The block is saved

without a header, as a normal data block (A, the flagbyte, is

0FF)

L: Load.  Loads a block of data from tape, at the specified

address.  Normal data blocks, headers and blocks with non-

standard flag bytes can be loaded.  The first byte in memory

will contain the flag byte.  If the checksum isn't 0 after

loading, indicating a tape error, you'll hear a beep.

H: Header read.  Loads headers and displays the contents on

screen.

As you're reading this part, I assume you know something of machine

code.  Probably you would be interested in peeking into the software of

the SamRam, the Interface I, the Spectrum 128, the Disciple or the

Multiface 128.  You'll first have to move these roms in ram to be able

to look at them with the monitor.

The Interface I rom can be moved into ram by saving it to microdrive or

to the "b" channel, with SAVE *"m";1;"rom" CODE 0,8192 or SAVE *"b"

CODE 0,8192, and loading it back again at 32768 for instance.  You can

also put this small machine code routine at 23296 and run it: F3 21 0C

5B E5 21 00 00 E5 C3 08 00 21 00 00 11 00 80 01 00 20 ED B0 FB C3 00

07.  You ought to be able to disassemble this piece by heart.

The two SamRam roms are easy.  The first you don't need to transfer;

the monitor looks at the extended basic rom by default.  The second rom

can be moved to 32768 by typing *SAVE 0,32768,16384.  (The SAVE is not

the keyword SAVE!)

The first '128 rom, the one which is active at reset and contains most

of the new code, is moved up by typing SAVE!"rom"CODE 0,16384, then

LOAD!"rom"CODE 32768.  The other rom is most conveniently moved by

saving it to a .TAP file and loading it back again in ram.  To select

the SamRam type SPECTRUM first, and then switch the hardware without

resetting.

The Disciple and Plus D roms can be transferred to RAM by simply saving

them to disk and loading them back at say 32768.

The Multiface ROM is paged by an IN from 191, and paged out again by

INning from address 63.  Don't forget to disable interrupts in between,

I'm not sure whether the M128 has a well-behaved interrupt routine.

ROM is from 0-8191, RAM on top of that.

4.  THE SPECTRUM

4.1  The Spectrum

This emulator supports the Interface I, the Multiface 128, the DISCiPLE

and Plus D interfaces, and the Spectrum 128.  Many Spectrum users will

have no experience with them, so some comments may be useful.  On the

other hand, I don't think this is the right place to describe the

Spectrum Basic in full detail.  If you want to know it all, read the

official manuals!  Information on the Multiface 128 can be found in

section 2.12.

If you want to use Spectrum Basic, you will need the keywords.  You

could by the way now also use the Spectrum 128 Basic where you can type

the keywords in by full.

If you press ALT-F1 in the emulator, the Spectrum keyboard layout will

appear.  For completeness I include an alphabetical list of all

keywords and their key-combination.  In the list below, K stands for

Keyword mode, E for E-mode (type Shift-Alt of Shift-Ctrl to select

E-mode), S for Symbol Shift, and SE for Symbol Shifted (Alt/Ctrl)

E-mode: select E mode and type the letter while depressing Symbol

Shift.

Keyw.    Code | Keyw.    Code | Keyw.    Code | Keyw.    Code

ABS      E g    DRAW     K w    MERGE    SE t   SAVE     K s

ACS      SE w   ERASE    SE 7   MOVE     SE 6   SCREEN$  SE k

AND      S y    EXP      E x    NEW      K a    SGN      E f

ASN      SE q   FLASH    SE v   NEXT     K n    SIN      E q

AT       S i    FN       SE 2   NOT      S s    SQR      E h

ATN      SE e   FOR      K f    OPEN #   SE 4   STEP     S d

ATTR     SE l   FORMAT   SE 0   OR       S u    STOP     S a

BEEP     SE z   GO SUB   K h    OUT      SE o   STR$     E y

BIN      E b    GO TO    K g    OVER     SE n   TAB      E p

BORDER   K b    IF       K u    PAPER    SE c   TAN      E e

BRIGHT   SE b   IN       SE i   PAUSE    K m    THEN     S g

CAT      SE 9   INK      SE x   PEEK     E o    TO       S f

CHR$     E u    INKEY$   E n    PI       E m    USR      E l

CIRCLE   SE h   INPUT    K i    PLOT     K q    VAL      E j

CLEAR    K x    INT      E r    POINT    SE 8   VAL$     SE j

CLOSE #  SE 5   INVERSE  SE m   POKE     K o    VERIFY   SE r

CLS      K v    LEN      E k    PRINT    K p    <=       S q

CODE     E i    LET      K l    RANDOMIZE K t   >=       S e

CONTINUE K c    LIST     K k    READ     E a    <>       S w

COPY     K z    LINE     SE 3   REM      K e

COS      E w    LLIST    E v    RESTORE  E s    DEC      SE q

DATA     E d    LN       E z    RETURN   K y    HEX      SE w

DEF FN   SE 1   LOAD     K j    RND      E t

DIM      K d    LPRINT   E c    RUN      K r

Character  Spectrum kbrd   On PC keyboard

&          S 6             ALT (or CTRL) 6

'          S 7             ALT 7 or '/"

(          S 8             ALT 8

)          S 9             ALT 9

_          S 0             ALT 0 or SHFT _/-

<          S r             ALT r or SHFT </,

>          S t             ALT t or SHFT >/,

;          S o             ALT o or :/;

"          S p             ALT p or SHFT "/'

^          S h             ALT h

-          S j             ALT j or _/-

+          S k             ALT k or SHFT +/= or GREY +

=          S l             ALT l or +/=

:          S z             ALT z or SFHT :/;

?          S c             ALT c or SHFT ?//

/          S v             ALT v or ?//

*          S b             ALT b or GREY PRTSC/*

,          S n             ALT n or </,

.          S m             ALT m or >/.

4.2  The Interface I

If you want to use the microdrive, you'll need cartridge files.  The

emulator can create an empty cartridge file for you.  You have to

format it before you can use it.  Type

FORMAT "m";1;"name"

to format the cartridge currently in Microdrive 1 giving it the name

'name'.  Next, type CAT 1 to get a catalogue of the files on it (none

of course) and the number of kilobytes free.  You can save a file by

typing for instance

SAVE *"m";1;"screen"SCREEN$

Instead of SCREEN$ you can use all other expressions that are permitted

also when saving to tape, like LINE nnnn or CODE x,y etcetera.  To load

a file back from cartridge, you type (you guessed it)

LOAD *"m";1;"screen"SCREEN$

If the file doesn't exist or is of the wrong type you'll get the

appropriate error message.  To erase a file, type for instance

ERASE "m";1;"screen"

Note that no * is needed (or even permitted), and that only the name

should be given.  There's another way to create a file on a cartridge,

and that is by using a command like OPEN #3;"m";1;"name", and printing

to that stream.  You can use MOVE to move data from stream to stream,

but I'll not go into that --- it's not very much used anyway.

Instead of to the microdrive, you can also 'save to the RS232 link'.

For instance, type SAVE *"b"SCREEN$ (note: there's no name!) to save a

screen.  On the emulator you can send the output to the RS232 channel

to a printer (then SAVE *"b" is useless), to a file (can be useful) or

to the COM port (very useful if you connect a real Spectrum to the PC's

COM port!).  You can load the data back by typing LOAD *"b"SCREEN$ and

making sure the RS232 channel is fed with the right input (from a COM

port or a file).  See also section 2.13.

If you want to use the RS232 channel for printing, open stream 3 for

output to that channel by typing

OPEN #3,"b"

or

OPEN #3,"t"

The first will simply copy everything you send to stream 3 (using for

instance LPRINT or LLIST) to the RS232 channel; the second converts

CR's into CR/LF's, breaks off lines at 80 characters and translates

keywords into character sequences.  "t" is useful for LLISTings, but

not for anything else.

Useful extra commands: CLS #, to clear the screen and reset the

attributes to their reset defaults, and CLEAR # to do a CLS # and close

all currently open streams (discarding all data that may still be

buffered!)

The Interface I uses its own system variables.  At the first error

message you make (or RASP, or flashing question mark) and at the first

Interface I statement you execute, it inserts them automatically.  Some

programs will not run when the Interface I has inserted its system

variables.  So if you load a game from tape, reset the Spectrum first

and don't make an error typing LOAD "".  With a bit of exercise you

should be able to do this.

4.3  The DISCiPLE and Plus D Interfaces - Introduction

The DISCiPLE and Plus D were two disk interfaces for the spectrum

designed by M.G.T. (Miles Gordon Technology).  The first of these

interfaces was the DISCiPLE, this interface consisted of a disk

interface, microdrive network compatible interface, parallel printer

interface, 2 joystick ports (emulating kempston, cursor, and Sinclair 1

and 2).  The unit also had an inhibit button which disabled the

interface hardware (except the joystick ports), and finally a snapshot

button which when pressed stopped the computer to allow the program to

be saved to disk or the screen to be printed.  This was made by

Rockfort.

The Plus D was the second of the interfaces, this was a cut-down

version of the DISCiPLE, this interface only had a disk interface,

parallel printer interface and a snapshot button.

Both interfaces had a D.O.S. (Disk Operating System) which was partly

ROM (8K) and partly RAM (8K).  When the spectrum was turned on, the ROM

part of the D.O.S. was in control and whenever the command RUN was

issued the ROM tries to load up the RAM part of the D.O.S. from floppy

disk.  The advantage of this is that the D.O.S. can be upgraded without

having to change chips over (unless of course it was a major upgrade!).

Another advantage was that D.O.S. extensions could be incorporated or

replace other systems (see later).

With both interfaces, they extended the BASIC commands, but unlike the

microdrives and several other types of drives available, the DISCiPLE

and Plus D took up none of the spectrum's RAM, therefore it was the

most invisible of the disk systems available, not only that, but the

DISCiPLE and Plus D used the same type of disk drives as the BBC micro,

therefore disk drives were both cheap and widely available, also the

disks themselves were also standard, ie 5.25" (800K DS/DD), then later

3.5" (800K DS/DD).

The ROM of the DISCiPLE is supplied with the emulator, together with

two version of the D.O.S. pre-loaded in RAM.  The ROM of the Plus D

interface is not supplied, for copyright reasons.  The emulator will

only emulate a Plus D interface if it can find a file PLUSDROM.BIN in

the emulator's own directory.  If you have a Plus D yourself, you can

transfer the ROM in the following way.  Load the operating system from

a Plus D system diskette, and save the entire ROM, together with the

operating system, to disk by entering SAVE d1"rom" CODE 0,16384.  Then,

start the emulator and choose hardware mode Spectrum 48K + Disciple

(not Plus D of course), then type LOAD d1"rom" CODE 32768.  Finally,

press F10, X, S for Save Data block, set start address and length to

8000 and 4000 (hexadecimal) respectively, and save the ROM under the

name PLUSDROM.BIN.  If you re-start the emulator now, it will emulate

the Plus D too.

4.4  The DISCiPLE and Plus D Interfaces - The basic commands

There are several levels of commands that can be used, these range from

the most straightforward everyday use, to the more advanced, programmer

type commands.

I will first explain the most common commands, so that you can quickly

and easily access DISCiPLE and Plus D disks.  Where a 1 is used in the

following commands 2 could be used instead.  These commands are:-

RUN -   when no D.O.S. (system file) is loaded it will cause this to

be loaded. Otherwise it will just run the BASIC PROGRAM.  Please

note that with the emulator the system file is already loaded,

therefore this command is not required, although I have

explained it for completeness.

CAT 1 - will display a longhand catalogue of the disk drive selected.

The form of this catalogue is as follows:-

program no., program name, sectors used, file type,

file size

CAT * - will display a longhand catalogue of the currently selected disk

drive in the same form as described above.

CAT 1! - displays a shorthand catalogue of the disk drive selected.

This catalogue consists of a 3 column list of the filename of

the programs.

CAT *! - displays a shorthand catalogue of the current disk drive.

LOAD pn - p - letter p, n - number between 1 & 80.  This is the program

number of the file on the disk, the program number is the

number printed before the name in the longhand catalogue.

LOAD d1"name" - load from drive 1 the program called name

LOAD d*"name" - load from the current drive the program called name

LOAD d1"name" S - load an 48K snapshot from drive 1 called name

LOAD d1"name" K - load an 128K snapshot from drive 1 called name

LOAD d1;a$ - load from drive 1 the program whose name is held in the

string a$

LOAD d*;a$ - load from the current drive the program whose name is held

in the string a$

FORMAT d1 -  format the disk in drive 1

4.5  The DISCiPLE and Plus D Interfaces - More advanced commands

In the previous section I explained enough of the commands so that you

could get to use the floppy disks with DISCiPLE/Plus D software on. Now

I am going to explain the commands that the more experienced user and

those who want to do just a little bit more than just load the

programs.

First of all comes the simple commands of SAVE, MERGE, VERIFY and LOAD.

All of these commands are the same as rge tape versions except that you

have d1, d2 or d* after the LOAD, SAVE etc.  There are only two

exceptions to this rule, the first is when you have a string for the

filename, in this case the command becomes:-

LOAD d1;n$ ....... etc

LOAD d*;n$ ....... etc

The second exception is actually an extension.  When you save a code

block, you can actually get it to autorun when it is loaded by adding a

third parameter to the SAVE command, e.g. if you had a code block from

40000 to 45000 and the run address was 41023 and you wanted it to be

called testcode to drive 1, you would save it as:-

SAVE d1"testcode"CODE 40000,5001,41023

Now for the extensions.

Any sector on the disk may be loaded to any area of RAM from 16384 to

65535-512. The sector may also be loaded into the RAM of the interface,

however caution should be used at all times when doing this, as you may

destroy the operating system or cause it to behave irrationally.

The syntax of the command is:-

LOAD @n,tr,sec,add

Where:-

n   = drive number, ie 1 or 2, note * may not be used!

tr  = track number => 0->79=side 0, 128->207=side 1

sec = sector number => 1->10 normally, may not be 0!  If you

try and load a sector that is sector 0,

then the operating system will crash!

You may also save to disk in the same way, so if you wanted to write to

Track 5, Side 0, Sector 3 with code from 32768 to 33279 to drive 1 you

would type:-

SAVE @1,5,3,32768

To format a disk all you need to type is:-

FORMAT d1       or       FORMAT d2

This is not however as simple as it seems!  Although a disk formatted

on the DISCiPLE/Plus D will work with the PC it does not work the other

way round, i.e. a disk formatted on the PC with this emulator will not

necessarily work on the DISCiPLE/Plus D. This is because of differences

of the floppy disk controller chips.  With the PC, it puts a special

byte at the beginning of each track to tell the controller whether the

disk is Double Density or High Density, and the spectrum floppy disk

controller does not understand this byte and therefore will not read

the track!  If you find this otherwise, then please write and let us

know about it.

You can set some of the D.O.S. system variables using an extended POKE

command. The syntax is:-

POKE @address,value

The address is the BASE address of the system variables of the

interface concerned, the BASE address for each of the interfaces is

different, but the address you use in the POKE command is the same.

4.6  The DISCiPLE and Plus D Interfaces - The snapshot button

The snapshot button is a button which is used to stop the processor

from executing instructions and making it do something else. In the

case of the DISCiPLE/PLUS D the button can do 5 different tasks

initially. Later on I'll discuss how the fuctions can be altered so

that it can do other tasks. Once these tasks are carried out, the

processor is returned to it's original state and the program continues

from where it left off from.

There are 5 functions that can be carried out with the initial system

file.

To use the button, first you must get to a point in the program that

you want to use the button at, then press the button on the emulator

the snapshot button is F5 (NMI).  Once you do this the program will

stop and the border of the computer will flash, this may also be

accompanied with a buzzing noise on the speaker.  This is to tell you

that the snapshot button has been activated.  The following functions

can then be carried out:-

1   - Print screen to printer in black/white screen size

2   - Print screen to printer in grey scale A4 size

3   - Snapshot screen to disk in SCREEN$ format

4   - Snapshot program to disk in 48K SNAP format

5   - Snapshot program to disk in 128K SNAP format

SPACE - go back to program (ie if F5 was pressed by mistake)

The printer options will print out to EPSON compatible printers,

however the DISCiPLE code has been changed and it is now possible to

print to HPGL printers (e.g. Deskjet and Laserjet printers).

The Snapshot screen and snapshot 48K are very self explanatory and easy

to use.

The Snapshot 128K is slightly more complicated.  Once you select this

option the disk drive will start up then after a slight pause the

screen display may/may not change.  The border flashes again and you

have to press y or n depending on whether the screen display changed or

not.  If the screen stayed the same then press n, but if the screen

changed type y.  This is because the 128K spectrum has 2 screens, and

there is no way for the computer to determine which is being used by

itself, therefore it needs the user's help.  After you press y or n the

program will be saved onto disk.

Seeing it is possible for you to load programs into the DISCiPLE/PLUS D

RAM area it is possible to have these interfaces carry out other tasks.

The snapshot button code is located in RAM, so you can therefore load a

program into this part of the RAM and as soon as you press the snapshot

button it will execute the new program.  There is one problem however,

which is that the program must be written in assembly language and must

also be compiled for running in that part of RAM, also it must take

care of what it does because the SPECTRUM ROM is not paged in,

therefore ROM calls are not easily carried out.

There are several different commercial programs which use the snapshot

button to carry out different tasks, some of these are:-

Snapshot compressing programs

Debugging tools

Cheat finders for games

Programs to allow snapshots back onto tape

The list is endless, and if you are capable of writing a machine-code

program and have the relevant information, then you too could write

your own programs.

4.7  The DISCiPLE and Plus D Interfaces - The system variables

The system variables reside in the RAM of the interface, and cannot be

read of changed by using the normal POKE statement.  For access to

these locations the designer of the Disciple and +D ROM has implemented

the POKE @x,y command.  It changes the value at address (#0298+x) in

the interface's RAM.  Here are the system variables, with their

meaning.  Default values are between brackets.

Note that when y (the second argument) of a POKE @ command is larger

than 255, it is interpreted as a word, and it is poked at locations x

and x+1.  Handy for locations 14 and 16.

POKE @0: (7)   Border change mask when reading/writing disks

POKE @1: (80+128=208)   Number of tracks of drive 1. 128 means DS

POKE @2: (208) Same for drive 2

POKE @3: (6)   Step rate in ms (may not be lower than 6)

POKE @4: (1)   Network (0=off, 1=master, 2..10=assistant, 11..63=slave)

POKE @5: (80)  Printer line width (# of chars printed before CR is

inserted)

POKE @6: (0)   Control code conversion (0=on, 1=off)

POKE @7: (12)  Printer line feed when printing graphics (1/72th inches)

POKE @8: (1)   Number of Line Feeds (10) inserted after each CR (13)

POKE @9: (0)   Left margin: number of spaces printed after each CR

POKE @10:(0)   Convert some characters into graphics (0=off, 1=on)

POKE @11:(0)   Enable MGT Centronics printer interface (0=yes,

1=route to standard "p" channel, i.e. ZX Printer)

POKE @12:(word 0)     Unused

POKE @14:(word 0)     Basic extension call vector

POKE @16:(word #3b0)  Basic extension call vector (I think)

The default value of variable 10 was changed to 0.  Previous rom images

supplied with Z80 had @11 set to 1 (printer disabled) and @8 to 0

(don't add line feeds), but these were not Disciple/+D defaults, so

they were changed back in version 3.05.

4.8  The Spectrum 128

The main new features of the Spectrum 128 are its larger memory, that

can be used as a Ram drive in Basic, and music capabilities.

The Ram drive is accessed via the LOAD!, SAVE!, ERASE! and CAT!

commands.  They work as you would expect.  Examples:

SAVE !"name"SCREEN$

CAT!

LOAD !"name"SCREEN$

ERASE !"name"

The 3 channel sound chip of the Spectrum 128 can be used in Basic with

the PLAY command.  Example:

PLAY "cde","efg","gAB"

plays three chords.  You can program complex effects, melodies and

rhythms with the play command; they require many commands in the three

voice strings which I won't explain...  They are explained in the

Spectrum 128's user guide.